======================================================

=                                                    =

=                                                    =

=          Parallel Port Information System          =

=                                                    =

=                   version 1.45                     =

=                                                    =

=                        by                          =

=                                                    =

=                     Jay Lowe                       =

=                    Don Schuman                     =

=                                                    =

=             Parallel Technologies, Inc.            =

=                                                    =

=                                                    =

======================================================

Version 1.45

27 Oct 1994

Reference Sites for Parallel

Compuserve Home:    IBMHW forum (go IBMHW)

Internet Home:      netlab2.usu.edu/misc

======================================================================

Parallel.Exe, v1.45, 27 Oct 1994           Parallel Technologies, Inc.

Parallel Port Information Utility                    10603 170th Ct NE

(C) Copyright 1993-1994 Jay Lowe                     Redmond, WA 98052

This program may be used                             Lab  206-869-1136

free of charge subject to the                        Fax  206-869-1133

Restricted License Agreement                  Compuserve     76640,203

as stated below                           Internet jaylowe@halcyon.com

======================================================================

Parallel Port Information System                                     Page 2

---------------------------------------------------------------------------

Front Matter

------------

IMPORTANT: This software is provided AS-IS under a license agreement

stated below (see SOFTWARE LICENSE AGREEMENT). YOU ARE RESPONSIBLE for

the safe usage of this program.  This requires that you READ THE

DIRECTIONS if you are going to use the loopback feature for IRQ

detection. We are informing you of the danger of physically damaging

your parallel port if you are not CAREFUL AND GENTLE when inserting

the loopback wire. We strongly recommend that you follow the procedure

of making a LOOPBACK EXTENSION CABLE as described below. To the best

of our (extensive) knowledge, there is no electrical danger to

yourself or to your system or to the parallel port, since parallel

ports are designed to withstand permanent shorts across any of their

pins. HOWVER WE DO NOT GUARANTEE AN OF THIS AND CAN ACCEPT NO

LIABILITY since we cannot control what you or your vendor may do.

Parallel.Exe, Parallel.Doc, and all of the associated components of

this utility are copyrighted material and may not be reproduced,

redistributed, or sold without the express written consent of Parallel

Technologies, Inc, except as provided by the License Agreement.

Some of the terms, names, and/or symbols used in this document are the

trademarks of the companies which own them.

Permission is hereby given for distribution of Parallel on a not-for-

profit basis.  Users may upload and share Parallel, provided it is not

modified in any way.  Vendors may make Parallel available via bulletin

boards or other electronic forums, but may not distribute Parallel

with their products without the permission of Parallel Technologies,

Inc.  Shareware vendors and other software distributors may distribute

Parallel provided they do not charge more than $10 per copy.

Support for Parallel and parallel port related issues is available

only via Compuserve or Internet Email to the addresses given on the

cover page of this manual.

Parallel Technologies, Inc. does not provide telephone support for

this program.

Parallel Port Information System                                     Page 3

---------------------------------------------------------------------------

Acknowledgements

----------------

We would like to express our appreciation to the many people and

organizations who have given of their time and expertise to provide

information and support for Parallel in various ways.  Some of these

contributors include:

Parallel Port Chipset Vendors:

Intel Corporation

Standard Microsystems Corporation

National Semiconductor

Chips and Technologies

Parallel Port Direct Sales Vendors:

Farpoint Communications

JAZ Computer Products

Essential Data, Inc.

Standards Organizations:

Members of the EPP Standards Committee

Members of the IEEE 1284 Standards Committee

Individuals:

Larry Stein, Farpoint Communications

Prof. Joe Dupnik, Utah State University

Don Watson, Compuserve IBMHW Forum Sysop

The users:

who have reported test results, bugs, and feature requests!

And the innumerable hackers who have gone before.

My thanks to all of you for your generosity.

And if I've left anyone out, my apologies. Please remind me.

Parallel Port Information System                                     Page 4

---------------------------------------------------------------------------

Table of Contents

-----------------

FRONT MATTER                                                         2

ACKNOWLEDGEMENTS                                                     3

TABLE OF CONTENTS                                                    4

WELCOME TO THE PARALLEL PORT INFORMATION UTILITY                     7

QUICK STARTING PARALLEL                                              8

DETAILED PROGRAM OPERATION                                           9

How to Prepare Your System for Best Results                         9

How to Run Parallel.Exe                                             9

How to Get Quick Help                                               9

How to Prevent Scrolling                                            9

How to Save Parallel's Output                                       9

How to Avoid the Funny Line Drawing Characters                      9

How to get Help with your Parallel Port Questions                  10

INTERPRETING PARALLEL'S DISPLAY                                     11

Parallel Port Type                                                 11

No Port                                                           11

4bit ports                                                        12

8bit ports                                                        13

EPP ports                                                         14

ECP ports                                                         16

IO Address                                                         18

Understanding the IRQ Level                                        20

Understanding the BIOS Name                                        22

Understanding the Notes and Comments                               24

Parallel Port Information System                                     Page 5

---------------------------------------------------------------------------

AUTOMATIC IRQ DETECTION                                             30

Understanding Port Float                                           30

IRQ detection using the LoopBack Feature                           31

TechNote: Making Your Own Loopback Connector                       33

MISCELLANEOUS TOPICS                                                34

Technote: Pseudo-Bidirectional Ports, A Special Case               34

PARALLEL PORT STANDARDS                                             35

4bit and 8bit parallel ports                                       35

EPP Specification                                                  35

ECP Specification                                                  35

IEEE 1284 Specification                                            36

Parallel Port Standards Diagram                                    37

PARALLEL PORT BUYER'S GUIDE                                         38

4bit parallel ports                                                38

8bit parallel ports                                                38

EPP Parallel Ports                                                 39

ECP Parallel Ports                                                 41

Advanced Parallel Port Chipsets                                    42

VENDOR CONTACT INFORMATION                                          44

TECHNICAL SUPPORT                                                   45

ABOUT PARALLEL TECHNOLOGIES, INC.                                   46

Firsts, Bests, and Mosts                                           46

Partial List of Parallel Technologies' Developments                46

Parallel Port Developer's Kits                                     47

Level One Developer's Kit                                         47

Parallel Port Information System                                     Page 6

---------------------------------------------------------------------------

Level Two Developer's Kit                                         48

Super IEEE 1284 Developer's Kit                                   48

DirectNet -- The Parallel Port Networking System                   48

Parallel -- the Parallel Port Information Utility                  49

OEM Products                                                       49

Development Services                                               50

Consulting and Training                                            50

SOFTWARE LICENSE AGREEMENT                                          51

Parallel Port Information System                                     Page 7

---------------------------------------------------------------------------

Welcome to the Parallel Port Information Utility

------------------------------------------------

Parallel is a simple utility which displays the configuration of the

parallel ports (LPT ports) in your system. Parallel is useful for:

1.  Configuring parallel port software such as Windows printer

connections, parallel port device drivers, and print spoolers.

2.  Resolving technical support problems with parallel devices,

including printers and external peripherals such as network

adapters, portable disks, tape drives, CD-ROMs, floppies, etc.

3.  Configuring your parallel ports for best performance.

4.  Resolving pesky IO address and IRQ contention problems.

5.  Measuring the performance capabilities of your parallel ports.

6.  Learning how your particular system's CMOS configuration

relates to the actual configuration of your parallel ports.

7.  Making informed decisions when buying parallel ports and

external parallel port devices.

8.  Getting the best parallel port performance for your dollar.

9.  Experimenting with your parallel port configuration.

10. Solving problems associated with print spoolers and network

print capture programs which redirect printer output by

changing the port mappings kept in the LPT Bios data area.

Now is a good time for you to learn something about parallel ports.

Parallel ports are becoming increasingly useful due to the recent

explosion in the popularity of portable and external peripherals.

New parallel port standards (such as EPP and ECP ) are breaking

performance barriers and allowing full bus speed operation.

Parallel ports have traditionally been perceived as "printer ports",

which explains a lot about their idiosyncrasies. In fact, a relatively

small percentage of parallel ports are actually used for printing. The

ratio of PCs to printers is usually "many to few" in a group setting,

and as networking and networked printers become increasingly common

this trend will increase. In general, PC parallel ports are usually

available for other purposes ... for portable, external peripherals

and parallel port networking (Desktop and Personal Area Networking).

There are major performance differences between different types of

parallel ports and parallel port devices, and these differences are

becoming of increasing significance to consumers. These applications

of Parallel are discussed in more detail below. But first let's take a

quick look at how you run the program itself and interpret the various

information it displays.

Parallel Port Information System                                     Page 8

---------------------------------------------------------------------------

Quick Starting Parallel

-----------------------

Parallel operates by testing each of the standard addresses in your

system where parallel ports might be found and gathering a broad

assortment of useful information about the specific port behavior.

Parallel tests the ports and then produces a compact display which

is convenient and easy to read. You may need to refer to these

instructions or the built-in reference screen a few times until you

learn what the various columns of information mean.

How to use Parallel:

Step 1 =>   Disconnect any attached parallel port peripherals such

as printers, LAN adapters, disks, CDROMs, etc

Step 2 =>   Run PARALLEL from the DOS command line (not in Windows)

Step 3 =>   Read the display as described below.

Parallel will display five columns of information containing one or

more lines about each port. The columns are:

Port Type         The parallel port's IO capabilities. Types are:

No Port   -- No port is exists at the address

4 bit     -- A "unidirectional" port

8 bit     -- A "bidirectional" port

EPP       -- An Enhanced Parallel Port

ECP       -- An Extended Capabilities Port

Address           IO Base Address of the port (3BC, 378, 278)

IRQ               Interrupt level associated with the port (5 or 7)

BIOS Name         Logical device name of the port (LPT1, LPT2, LPT3)

Notes             A wide assortment of comments, warnings, and

miscellaneous additional information

Port              On a single line, after the columnar display,

Strangeness       Parallel ports a magic number called the

Factor.           `Port Strangeness Factor'.

This is simply the total number of strange or unusual properties that

Parallel has found when examining your configuration. If your ports

are considered strange by Parallel, you might want to take some action

to correct the situtation, or at least appreciate the peculiarity.

The Port Strangeness is not reported if your configuration is boring.

Detailed procedures for using Parallel are given in the next section.

Parallel Port Information System                                     Page 9

---------------------------------------------------------------------------

Detailed Program Operation

--------------------------

How to Prepare Your System for Best Results

-------------------------------------------

Before running Parallel, temporarily remove all parallel port

devices and software to establish a baseline configuration.

Temporarily disconnect printers and other parallel port devices

such as disks, network adapters, tape drives, etc, and disable any

special parallel port software you may have running, like print

spoolers or parallel port device drivers for external devices.

The interaction of Parallel with external devices is discussed in

some detail later on, and you may find that disconnecting these

devices is not necessary in your configuration.

How to Run Parallel.Exe

-----------------------

Type PARALLEL at the DOS command line.

Parallel will display the parallel port type, IO address, IRQ

level, BIOS names, and an assortment of informative notes about

your parallel ports. Running Parallel in a Windows DOS box

generally provides useless results.

How to Get Quick Help

---------------------

PARALLEL ?               ; displays the a reminder screen

How to Prevent Scrolling

------------------------

If, perchance, the display scrolls off the screen, which might

happen if Parallel is reporting extensive notes about interesting

conditions on your ports, then use the MORE command as follows:

PARALLEL | MORE          ; pauses the output every screen

How to Save Parallel's Output

-----------------------------

Use DOS redirection to write Parallel's display to a file:

PARALLEL > filename      ; redirects output to a file

How to Avoid the Funny Line Drawing Characters

----------------------------------------------

Sorry, you can't.  We plan to add a switch to omit or replace the

linedrawing characters for more convenient display via email, etc.

Parallel Port Information System                                    Page 10

---------------------------------------------------------------------------

How to get Help with your Parallel Port Questions

-------------------------------------------------

1. Contact your vendor.

Your vendor has a responsibility to support you in using

their products.

If you have a problem with a parallel port device, your

vendor may already know a successful resolution.

Some vendors maintain BBS systems or Compuserve forums

on which may occasionally be found useful information,

utilities, and driver software upgrades.

Many vendors of parallel port peripherals offer very

good technical support IF YOU GIVE THEM A CHANCE ... or

several chances!  Don't hesitate to be persistant.

If you have a problem configuring a PC parallel port,

your vendor may already have a fix ready for you. This

may consist of a utility program, a BIOS upgrade, or a

manual procedure for configuring the port.  Since PC

vendors use a bizarre array of different methods for

parallel port configuration, your best bet is really

the vendor themselves. Alas, however, some PC vendors

are completely clueless about parallel ports, although

this situation is improving rapidly.

Give your vendor a chance to resolve the problem. They are

sometimes the most responsive source of assistance, AND THEY

NEED YOUR INPUT.  Let them know what you want, and what you

are doing -- it helps them adjust their plans and priorities.

If you are not getting the proper assistance, call your

vendor and let the product manager know about the situation.

Think of it as your contribution to overall product quality!

And if Parallel doesn't detect your ports tell your vendor to

(a) get a copy of Parallel and (b) help us detect their port!

2. Post your question in the IBMHW forum on Compuserve or in the

IBM PC Hardware newsgroup on the Internet.

Quite a few technical gurus read these forums and they tend

to be generous in giving you the benefit of their experience.

3. Email to CIS ID 76640,203 or jaylowe@halcyon.com.

We can't promise immediate service (or indeed any service at

all!) but we try to do our best.  Please understand that we

get quite a few random technical inquiries and have to fit

them around our development schedule.

Parallel Port Information System                                    Page 11

---------------------------------------------------------------------------

Interpreting Parallel's Display

-------------------------------

Parallel Port Type

------------------

Parallel reports the port type as 4bit, 8bit, EPP, ECP or No Port.

No Port

-------

No port was found at the given IO Address.  This is a

normal result if no parallel port is installed at the

given address.

Parallel tests all the base addresses at which parallel

ports are commonly found and displays at least one line

of information about each base address, even if there

is no port installed at that address.

Parallel uses a rather more robust technique for

detecting a port than is used in any of the BIOSes we

have seen. This means that Parallel can detect ports

which fail the BIOS boot-time tests. Parallel uses a

rotating bit test and assumes the port exists if any of

the bits at the port location respond correctly.

Parallel is also starting to detect specific advanced

parallel port chipsets. Intel and SMC detection is

present in this release and National and C&T detection

will be included in the next release.

False Positives.  Parallel can be fooled into thinking

that a port is present when it is not (false positive)

only if there is some other device which behaves like a

parallel port located at the address being tested.  We

have only seen this on one system, an off-shore 286

laptop which had several other design problems as well.

False Negatives.  There is one way in which Parallel

can be fooled into thinking that a port does not exist

(false negative). If an active device is connected to

the data lines and is providing enough power to

"overdrive" the port, then the port existance test will

fail because the incoming data from the device will

spoof the readback test. We have only seen Parallel

produce false negatives when an external device is

actively driving the port's inputs or when the Loopback

connector is incorrectly installed. By the way, in this

case, the PC's BIOS will also fail to detect the port.

Remember, Parallel works best when you disconnect any

external parallel port devices or cables.

Parallel Port Information System                                    Page 12

---------------------------------------------------------------------------

4bit ports

----------

The port can do 8bit byte output and 4bit nibble input.

These ports are often called "unidirectional", and are

most commonly found on desktop bus cards (also called

IO expansion cards, serial/parallel cards, or even 2S+P

cards) and older laptops. This is still the most common

type of port, especially on desktop systems.

HOW FAST:  4bit ports are capable of effective transfer rates of

about 40-60 KBytes/sec (KBPS) in typical devices and

can be pushed upwards of 140 KBytes/sec with certain

design tricks. The port speed is also affected by

electrical characteristics of the noise suppression

circuitry. A few ports have completely ridiculous

arrangements of noise suppression circuits which slow

them down quite dramatically and can prevent their

correct functioning with parallel port peripherals.

TECH NOTE:  The sample transfer rates given here reflect the actual

performance of parallel port network adapters, disks,

etc. The raw transfer rate of the port itself is higher

but since most parallel devices use software strobing

to access data , the effective transfer rate is

dependent on the quality of the software supplied with

the device. Due to the recent explosion of parallel

port peripherals entering the market, there is an

extreme degree of variability in the quality and

performance of these devices and their drivers.

WHERE FOUND:  4bit ports are found on about 50% of all systems and

about 75% or more of desktop systems prior to 1993.

Systems built after 1993 are more likely to have more

capable parallel ports such as 8bit, EPP, or ECP.

TECH NOTE:  Many new systems, especially laptops, have parallel

ports whose mode can be selected via a CMOS setup

procedure.  Often these system default to a 4bit port

type even if more port capabilities are present.

We do not recommend 4bit ports; 8bit ports are better.

TECH NOTE:  There is a class of "semi-bidirectional" parallel ports

which appear to the PC to be 4bit ports, but actually

have 8bit capability when they are connected to

external devices with strong drivers. These include the

original PC/AT parallel port. Parallel will report such

ports as 4bit ports but they may actually function as

8bit ports with better parallel port peripherals which

are especially designed to use such port as 8bit.

Parallel Port Information System                                    Page 13

---------------------------------------------------------------------------

It is actually possible to identify this type of port

using a side-effect of Parallel's loopback test. See

the special section below on Semi-Bidirectional ports.

8bit ports

----------

These ports can do both 8bit input and output.

These ports are sometimes called "bidirectional ports",

but that term is often misused by some vendors to refer

to 4bit ports as well. Most newer laptops have 8bit

capability although it may need to be enabled with the

laptop's vendor-specific CMOS setup function. This is

discussed below. A relatively smaller percentage of LPT

bus cards have 8bit capability which sometimes must be

enabled with a hardware jumper on the board itself.

True 8bit ports are preferable to 4bit ports because

they are considerably faster when used with external

devices that take advantage of the 8BIT capability.

HOW FAST:  8bit ports are capable of speeds ranging from 80-300

Kbytes/second, again depending on the speed of the

attached device, the quality of the driver software,

and on the port's electrical characteristics.

WHERE FOUND:  8bit ports are found on about 25% of desktops, about

50% of laptops, and 90% of laptops built since 1991.

CAVEAT EMPTOR: A number of vendors claim "bidirectional" capability

for their parallel ports when they are actually mere

4bit ports. When buying a parallel port expansion card,

or an entire system, you might want to use Parallel to

find out the REAL capabilities of the port. Be aware

that a vendor's general reputation for quality may not

translate into good parallel ports.  There are examples

of well-known vendors making false claims for their

parallel ports... sometimes, it appears, simply because

they don't understand them... the claims or the ports.

We are not going to pan anyone's "bidirectional"

parallel ports specifically, but be warned that the use

of terms like "bidirectional" or "PS/2 compatible" may

have nothing to do with the port's actual capability.

Some vendors even refer to 8bit ports as "enhanced",

which is totally misleading and creates no end of user

confusion. Make sure exactly what you are getting.

The moral of the story ... use Parallel to test ports

to determine their real I/O behavior and compatibility.

Parallel Port Information System                                    Page 14

---------------------------------------------------------------------------

Remember that if Parallel can't detect the port, or

reliably report it's actual behavior, then chances are

good that other software won't be able to either!

EPP ports

---------

Can do both 8bit input and output at ISA bus speeds.

This type of port is defined by the EPP 1.7 standard,

by the actual behavior if the Intel 360SL I/O chip, and

also by the IEEE 1284 standard (which is substantially

incompatible with the EPP 1.7 standard - more below).

HOW FAST:  These ports are as fast as 8bit bus cards and can

achieve transfer rates of upwards of 1 Mbyte/sec. For

example, our EPP-capable external SCSI disk is faster

on a popular SL laptop than it's own internal drive.

EPP ports are supposed to conform to a well defined

electrical standard and many of them actually do so.

However, if you are buying a laptop and want to be sure

of using fast EPP-capable external peripherals, then

you should definitely arrange to try before you buy.

HISTORY:  EPP ports came about as a joint effort involving

Zenith, Intel, and Xircom.  After the introduction of

the EPP port capability in the Intel 360SL I/O chip,

many manufacturers began providing this feature on

their SL-based laptops. An EPP Standards Committee was

formed which standardized the behavior of the Intel SL

EPP port as the EPP 1.7 standard ... and life was good.

Then the EPP standards committee merged with the IEEE

1284 committee which was itself struggling to specify

new parallel port type(s) with a predominant focus on

fast and bi-directional communication for printers.

Parallel port peripherals other than printers were

considered strange beasts and received extremely short

shrift in the 1284 deliberations, to the point that the

1284 specification does not really embody a long term

vision of the parallel port as a high-speed external

pathway for interfacing various peripherals to the PC.

Also, the 1284 committee did not like the details of

how the signalling occurred between the PC and the

attached device in the EPP 1.7 spec, and agreed to

include EPP within the scope of IEEE 1284 only if the

EPP standard was changed to correspond to the rules for

1284 signalling.  Hence the IEEE 1284 EPP spec, which

is incompatible with the original EPP 1.7 spec.

As a result, there are actually TWO standards for EPP

signalling, the original 1.7 specification (embodied in

Parallel Port Information System                                    Page 15

---------------------------------------------------------------------------

hundreds of thousands of SL laptops) and the IEEE 1284

EPP specification (embodied in a new generation of IEEE

compatible parallel ports which support multiple modes

of parallel port operation - see below). A typical IEEE

1284 style parallel port supports a combination of

Compatible (4bit and/or 8bit), EPP (version 1.7 and/or

1284) and/or ECP (which we visit in the next section).

EPP BIOS:  To make matters more complicated, the EPP Standards

Committee began work on an EPP BIOS spec quite some

time ago but the specification is still not adopted

either formally by the (now) IEEE 1284 Committee or by

any significant numbers of vendors. It is uncertain

whether such a BIOS will ever be widely supported, and

in any case, it will be too little - too late, because

developers will still be required to support the

existing defacto standards for non-BIOS interfaces as

well as the hypothetical new BIOS standard. This

reduces the EPP BIOS's usefulness; instead of being a

global standard, it becomes just another special case.

If this seems complicated to you, then congratulations,

you have successfully evaluated the situation!

CAVEAT EMPTOR: Therefore, when buying a system or external peripheral

which has high-speed EPP capability, it is crucial to

determine whether the system or device supports EPP 1.7

or EPP 1284 or both.  See the Buyer's Guide section for

more information on this subject.

Xircom was active in creating and defining the EPP 1.7

standard, and since their EPP-capable network adapters

are fairly common, it's worth mentioning that these

adapters can be used as a practical benchmark for the

goodness of a EPP 1.7 port. Crude but effective.

EPP ports are most often found on laptops using the

Intel 386SL chipset with the 360SL I/O support chip or

a register-compatible parallel port implmentation. You

can usually identify these laptops by the 'SL' suffix

in their name, HOWEVER this is not a hard and fast

rule, since there are non-SL systems with EPP ports as

well as SL systems which (in our tests) have non-

functioning or sub-standard EPP ports. Other vendors

offer add-in EPP cards for the ISA bus.

The Future:  Standard Microsystems Corp., National Semiconductor,

and Chips and Technologies have released multifunction

chipsets with both EPP and ECP capability, so users can

expect to see fast ports become available on a wider

range of systems. Other such chipsets are on the way.

This new generation of chipsets, combined with the IEEE

1284 standard, provides another window of opportunity

Parallel Port Information System                                    Page 16

---------------------------------------------------------------------------

for a comprehensive parallel port technology solution.

It remains to be seen if parallel port hardware and

software vendors will produce a cohesive response, or

if parallel ports will remain the PC's prodigal child.

ECP ports

---------

Can do both 8bit input and output at bus speeds.

The specification for this port type was jointly

developed by Microsoft and Hewlett-Packard.

ECP ports are distinguished by having DMA capability,

on-board FIFOs at least 16 bytes deep (too small), some

hardware data compression capability, and are generally

more featureful than other ports.

The MS/HP ECP port specification defines the ISA bus

registers by which the PC software can read and write

data at high speed to an attached device.

The MS/HP ECP port specification (what's inside the PC)

is related to the IEEE 1284 specification (what happens

on the wires outside the PC in various modes).

HOW FAST:  These ports are as fast as 8bit bus cards and can

achieve transfer rates upwards of 1 Mbyte/sec and

faster on PCs whose buses will support it. The design

is capable of faster transfer rates in the future.

TECH NOTE:  ECP port specification defines an ECR control register

with port mode configuration bits which may be used to

place the ECP port into several different submodes.

The ECP specification defines a multimodal port which

can support backward compatible and enhanced modes of

operation.

My goodness, this is starting to look like an excellent

solution to the proliferation of incompatible parallel

port standards. A standard port with multiple modes!

Alas, the ECP port specification does not define an EPP

mode, so vendors have chosen various ways of providing

an EPP capability within the framework of the ECP port

specification. Let's look at how this works.

SMC, for example, uses a reserved bit of the ECR

register to place the port into EPP 1284 mode on their

66xQPF chip (see Parallel Port Buyer's Guide). The

66xQPF chip, used on the Intel Pentium motherboards,

does not support EPP 1.7, which means that the large

installed base of parallel port network adapters,

Parallel Port Information System                                    Page 17

---------------------------------------------------------------------------

external disks, SCSI adapters, etc generally don't work

on such systems. When SMC figured this out, they

announced the 66xGT revision of the chip, which has TWO

EPP modes, 1.7 and 1284, as well as various bug fixes.

In addition, the SMC chip implements a TWO level

approach for controlling parallel port configuration.

The 66x contains configuration registers which ENABLE

the port to be in Standard, EPP-only, ECP-only, or EPP-

ECP modes. If the port is in ECP or EPP-ECP modes, then

the actual submode is SELECTED according to the ECP

spec by using the ECR register which appears on the bus

only when the chip is ENABLED for ECP or EPP-ECP.

To make matters even more complicated, other vendors

have taken other approaches to shoehorning EPP into the

framework of the ECP specification.

STANDARDS:  Note that while both the EPP and ECP parallel port

specifications establish how the port operates within

the PC when it is enabled, the specs do not describe

how the enablement occurs - that is left to vendors.

For example, the EPP 1.7 specification defines the

various I/O registers of the EPP port, but does not

define how to detect such a port or enable it.

Similarly, the ECP port specification defines the port

I/O registers and some of the configuration processes,

but does not define exactly how such ports are enabled,

this is a constant problem with inadequate standards.

Therefore it is possible for vendor ABC's excellent EPP

1.7 (or ECP, etc) device to fail to operate on vendor

XYZ's system because XYZ implemented the port in a way

that ABC does not recognize (yet). Parallel port device

vendors are constantly challenged by the appearance of

new types of ports - and the better vendors frequently

have software upgrades which support specific ports.

Existing parallel port standards have missed the mark.

they do not qualify as fully conceived response to the

overall requirement for speed, vendor flexibility, and

ease of use in the PC environment. These standards have

focused on bringing short-term capabilities to market

rather than on providing a comprehensive, universal

solution to the end user.

In the Buyer's Guide section, we will attempt to throw

some light on your practical options when shopping for

a system with fast parallel ports. The Parallel Port

Standards section reviews the various parallel port

standards, de facto and de jure.

Parallel Port Information System                                    Page 18

---------------------------------------------------------------------------

IO Address

----------

Also called the "base address" the port. It will almost always be

3BC, 378, or 278 hex. A very rare subset of parallel ports may

support additional addresses such as 280, 290, etc.

The IO address specifies the port's location on the PC's IO bus.

Normally the first port is configured to 3BC, the second to 378,

etc, however ports can be assigned to any of these three addresses.

On bus cards, the IO address is usually set by a arrangement of

jumpers unique to the particular board. You will need a manual or

the ability to read PC board traces (gacck!). These jumpers are

usually arranged to allow you to select one of three addresses.

Sometimes these addresses are called by the "logical device names"

of LPT1, LPT2, and LPT3, although this practice is quite misleading

(see BIOS discussion below).

Some boards offer a more limited selection of configurations,

usually 378 and 278, and a few are actually hardwired to a specific

address. Vendors do this to save a little money and to eliminate

the chance of creating an address collision with the "standard"

address of 3BC. Because of the increasing number of clones which

are assembled from various components, we often see systems with

one LPT card, configured to address 378. In fact, a fair number of

systems are shipped with this 378 default configuration.

A slightly interesting result we've seen in several desktops spins

off when someone adds a second LPT card to a system which already

has a LPT at 378, IRQ 7. Sometimes they naively reason that since

378 is taken they can use 3BC and slap in the card without checking

the IRQ level. This creates a configuration of 3BC and 378 both

using IRQ 7, a situation which can lead to intermittent crashes

when using interrupt-driven parallel port software.

By the way, some systems have parallel ports and com ports built

onto the motherboard itself and, in this case, the configuration

jumpers will be found on the motherboard or, as in the case of more

recent systems, in the CMOS SETUP utility.

The new generation of ECP-capable parallel ports typically

implement software selectable modes, base addresses, and IRQs.

If you have one of these new parallel ports, you will require

appropriate software in order to configure your port.  This may

exist within the CMOS setup function or may exist as a separate

utility.  When you buy a system or an add-in card which has

advanced parallel port functionality MAKE SURE YOU GET THE SOFTWARE

which allows you to enable it. Some vendors include it, some don't.

Parallel Port Information System                                    Page 19

---------------------------------------------------------------------------

Be aware of the distinction between configuration software which

allows you to control the configuration of your port, and device-

specific driver software which uses the port.

Notice that there are three commonly used IO Addresses (3BC, 378,

278) and two available interrupt levels (IRQ 7 and IRQ 5). This

means that if you have three LPT cards in a system, two of them

will probably be configured to share a hardware interrupt level.

This is hardwarily permissible, and generally doesn't create any

problems since most DOS and Windows software doesn't ENABLE

interrupts for printing anyway. However, if you are using LPT

software like an interrupt-driven print spooler or interrupt-driven

LPT devices like external hard disks, external network adapters,

and so forth, then you have to be sure that either (1) the ports

are not using the same interrupt level, or (2) both the ports and

the software are capable of "interrupt sharing".

When adding or configuring LPT cards, we suggest using 3BC, IRQ 7

for the first card; 378, IRQ 5 for the second card; and 278, no IRQ

for the third card. This gives you two interrupt-capable LPT ports,

and one port (at 278) which you should use only for non-interrupt

driven usage, like the status-driven spooling provided by DOS.

You should only configure your system to share interrupts when you

are (1) absolutely sure that they will not be simultaneously

enabled, and (2) when you have totally run out of other options.

Also, you should know that many sound cards, like the SoundBlaster,

arbitrarily use IRQ5 by default. This was a bad design decision,

especially so since the SoundBlaster's implementation of interrupts

creates a serious conflict when sharing the interrupt with other

devices, but we are stuck with it.  If you are using a sound card

on IRQ 5, then you should definitely AVOID ENABLING that level on

any of your parallel ports.

Armed with this understanding and the Parallel Utility, you are

equipped to select a configuration which works for you.

HOW TO:  The IO ADDRESS is a standard parameter in network

configuration files where it takes on different names

to confuse innocent users. (Base, Port, etc).

Some simple-minded parallel port peripherals (ones

without auto-configuring capability) require that you

specify the IO address, IRQ level, and sometimes the

Port Type in a vendor-specific way. Parallel provides

you with this information.

HOW TO:  Resolving IO Address conflicts is usually fairly

simple, since it is rare to find a device which uses

the standard parallel port addresses.

A good procedure for eliminating potential IO address

conflicts when installing parallel port add-in boards

Parallel Port Information System                                    Page 20

---------------------------------------------------------------------------

is to remove all the boards, install one, use Parallel

to see what address it finds, install the next, and so

forth. If Parallel finds each board at a separate

addresses as you install them, then you can be sure of

your configuration.

Understanding the IRQ Level

---------------------------

This is the hardware interrupt level assigned to the port. It will

almost always be either 7 or 5. Only a few specific parallel ports

support IRQ levels other than 7 or 5.

Parallel displays:

"?" when it cannot detect the IRQ level of the port and

"-" when it knows the the port doesn't exist.

Parallel needs an external loopback connector to automatically

detect the IRQ. With appropriate precautions (described in horrible

and excruciating detail below) you can use a simple paperclip as a

loopback connector.

Normally IRQ 7 is assigned to the first port and IRQ 5 is assigned

to the second port. However, due to the explosion of clones

assembled from disparate parts, it is not uncommon to see systems

with one LPT port located at 378 with an IRQ of 7 or 5. This is

because clones are often assembled using offshore add-in LPT cards

which are shipped configured to operate as a second LPT card.

This is a good time to point out that there are few hard and fast

rules when it comes to parallel port configuration. Ports can be

located at any one of the three standard base address and can use

either of the IRQ levels. If you are developing parallel port

software, you should auto-detect the actual configuration rather

than assume any standard usage.

We offer libraries and source code for this purpose. Here at

Parallel Technologies, we test our software using many strange

configurations and delight in adding idiosyncratic ports to our

extensive collection.

The most common idiosyncrasy of port IRQ is IRQ inversion. The PC

standard calls for interrupts to be generated on 0->1 transitions

of the interrupt line.  A small percentage of parallel ports

generate their interrupts on 1->0 transitions.  Ports with inverted

IRQ can fail to operate with some interrupt software, particularly

certain external network adapters, disks, and so forth. Parallel

identifies these ports with a note "IRQ style is INVERTED".

Some parallel ports have non-functioning IRQ lines. They are simply

not capable of generating interrupts. Only a very few pieces of

software are capable of dealing with these ports by using a polled

Parallel Port Information System                                    Page 21

---------------------------------------------------------------------------

mode of operation. It is sometimes possible to repair such systems

by jumpering the interrupt enable line to the parallel port chip.

In the case of an add-in board with broken IRQ capability, check to

be sure the IRQ jumper, if any, has not gone missing somehow.

On ISA cards (bus cards in ISA slots) the interrupt level is

usually set with jumpers on the board - these jumpers are different

for every card so you need to hunt around for the manual in order

to change them. If you're lucky there will be readable labels on

the card itself.

On laptops and an increasing number of desktops, the IRQ level can

usually be set using the vendor's CMOS configuration (setup)

program. Sometimes these CMOS setup programs are built into the

ROM; sometimes they exist as separate programs or as part of the

vendor's diagnostic program(s). Refer to your manual to find out

how to enter the CMOS setup mode. BE CAREFUL when using the CMOS

setup, since it gives you access to parameters that can disable

your system if wrongly changed. KNOW WHAT YOU ARE DOING. DON'T

CHANGE ANYTHING YOU DON'T FULLY UNDERSTAND. WRITE DOWN THE INITIAL

VALUES OF ANY SETTINGS YOU CHANGE. ASK YOUR VENDOR FOR HELP.

******************

* IMPORTANT NOTE *

******************

Misuse of your system's CMOS setup program can prevent your system

from operating correctly, or in some cases, from operating at all!

Be sure you know what you are doing; read your user's manual or get

qualified help if you have any doubts.

Another strange feature of these software configurable ports is

that they often have very peculiar ideas about nomenclature. Often

these CMOS setup programs use vendor specific "magic code words" to

select various parallel port configurations, and these magic words

can have non-obvious effects. Sometimes the vendor's manuals don't

even provide a secret decoder ring to explain what their magic

words really mean in terms of IO Address, IRQ, and Port Type.

Parallel will be useful for determining the actual effect of the

CMOS setup choices on your system.

For example, on my Dell NL25 laptop, 4bit mode is called "Normal"

and 8bit is called "Enhanced" and there is no choice at all for the

real Enhanced Parallel Port (the SL's EPP port). So someone might

obviously think they have enabled EPP by choosing "Enhanced".

Futhermore, the IRQ level cannot be set directly, but rather is set

as a by-product of selecting combinations of the IO address and the

Port Type. Most of the choices on my Dell yield IRQ 7, but one

lonely choice - LPT1, 278, Normal mode results in IRQ 5. It makes a

bizarre sort of sense when you are experienced with parallel ports,

Parallel Port Information System                                    Page 22

---------------------------------------------------------------------------

but is likely to be confusing to the Average User (if we ever find

him, or her).

The Dell is hardly alone in it's strange naming conventions ...

most PCs have confusing, misleading, or incomplete labels and

controls in their CMOS setup programs. This sort of indirect

control in very common in CMOS setups and creates a lot of tech

support calls from frustrated users.

In addition, most SL laptops don't have the capability to enable

their EPP ports from CMOS. Usually you need to run a separate

vendor-supplied BIOS extension to enable EPP capability. These take

the form of a tiny TSR which adds INT 17h BIOS extensions for

controlling the EPP port, and handles command line arguments for

enabling and disabling EPP. An interesting sidelight of switching

into EPP mode can be that your port IO address may change on the

fly. The Intel EPP hardware allows EPP ports to be configured at IO

Addresses 378 and 278. So if you have set your port address to 3BC

(with CMOS for example) and then switch into EPP mode, your port

will appear to have moved.

Parallel will come in handy in trying to make sense of what your

system's CMOS settings are actually doing to your parallel ports.

Understanding the BIOS Name

---------------------------

This is the "human-readable" (hah!) name which you use to refer to

the port when using DOS commands, Windows picklists, and so forth.

It will be one of the following: LPT1, LPT2, LPT3.  LPT4 is

extremely rare and Parallel currently ignores it.

The port address is NOT the same as the BIOS name for the port. For

example, LPT1 can refer to a port at any address: 3BC, 378, or 278.

So how does the BIOS associate the LPT names with the actual ports?

It works like this ...

Whenever your system boots, the BIOS goes out and tries to write

and read back data from locations 3BC, 378, and 278. If it can read

back what it writes, then the BIOS decides that a parallel port is

there and adds the IO address to a LPT table in the "Bios Data

Area". This table is four words located at memory location 40:8.

A command like "copy foo.txt lpt2:" really means "copy the file

foo.txt to the second printer that BIOS knows about" and DOS uses a

software interrupt, INT 17h, to do just that. INT 17h looks in the

Bios Data area to find out the IO address of the "second printer

port", which can be any of the IO addresses - 3BC, 378, or 278.

The result is that LPT1 refers to the first port BIOS finds at boot

time, wherever it may be. So "LPT1" means "the first port found in

Parallel Port Information System                                    Page 23

---------------------------------------------------------------------------

the address order", not necessarily 3BC, 378, or 278 specifically.

Some BIOSes have unique methods for finding their LPT ports.

When you see LPT1, LPT2, or LPT3 used to refer to port locations,

be alert for possible misleading information. Some well-known

vendor's manuals use LPT1 to refer to 378 and LPT2 to refer to 278,

etc, an entirely spurious practice which has confused many users.

It is possible for some parallel port software to set the port into

an IO mode that causes it to fail the simple BIOS test and seem to

"disappear" from your system even after a soft boot (CTRL-ALT-DEL).

Parallel has a better port detection test, and can usually find

such ports. A hard reset (a power-off reset) will clear the port.

Parallel ports are not completely trivial devices; they have a

number of states that can affect their operation. So if you are

having a parallel port problem, one of the first things to do is a

power-off reset.

Another factor here is that various programs can fiddle with the

LPT table in order to rearrange how the names relate to the ports

or to accomplish other effects like network print redirection.

The utility SWAPPRT, for example, can be used to swap LPTx with

LPTy which it does by moving the entries in the LPT table.

There are variations of this redirection theme.  Network print

capture functions (such as those founf in Netware and so forth) put

special values in the LPT BIOS table to mark that the port is not a

physical port but rather is a ``                                       virtual port'' which is actually

served by a software function, usually redirection of INT 17.

Some parallel port devices with passthrough printing ports also use

this technique to intercept the output of programs which print

using the INT 17 function call.

If Parallel does not display a BIOS name for a port, that means the

IO Address for that LPT position has been replaced with a value

that does not match a valid hardware port. This can occur when some

other parallel port software has grabbed possesion of the port by

placing a "magic number" in the BIOS LPT Table.

Remember, the BIOS decides how LPT symbolic names are mapped onto

physical port addresses, generally by a test a boot time, not the

vendor's manual, or the jumpers, or the CMOS settings.

Parallel Port Information System                                    Page 24

---------------------------------------------------------------------------

Understanding the Notes and Comments

------------------------------------

The "Notes" field of the display will contain various comments,

depending on what Parallel has detected. Each note may be followed

by one or more comments giving further information or directions.

These secondary comments are indented one space under their note.

"Port is an XYZZY chip"

Parallel is reporting the specific chipset used to

implement the parallel port at that address. Some

vendors may design completely compatible chipsets

(thank you) which spoof the specific chip detection.

Parallel may also report additional information about

the current setting of the chipset.

For example, in the case of an SMC 66x chip that is

enabled for EPP-ECP mode and which currently has the

Compatible submode of ECP selected (whew!), Parallel

will report an ECP port with a Note that the port is

currently in 4bit mode.

"No port found at this address"

No parallel port is installed at this IO Address.

This is a normal result if no port in installed there.

However, it is possible that something is connected to

the parallel port and is overdriving the port's inputs,

which prevents the software readback test from working.

Be sure you have disconnected all external devices.

"Need loopback to determine IRQ"

The port exists, but does not respond to attempts to

generate IRQs. This is because a loopback connector is

required for IRQ detection in most cases.

"Port IRQs are not functioning"

This is pretty bad. It means that the loopback

connector is present and working correctly, but

Parallel still can't generate any interrupts.

Technically it means that transitions in the ACK line

are not causing interrupts through the PC's interrupt

controller.

Parallel Port Information System                                    Page 25

---------------------------------------------------------------------------

In practice, it means that interrupt-driven devices and

software won't work, unless they have polling support.

Possible reasons for failing the IRQ test include:

1. The port's interrupt capability is disabled

Check your CMOS configuration and/or the hardware

jumpers on the parallel port board(s) to be sure

that the interrupt is enabled. The function of

CMOS parameters and/or hardware jumpers varies

from system to system, so check your manual or

contact your vendor if you have problems enabling

the port's interrupt line.

Note that most CMOS and jumper arrangements

combine enabling the IRQ with selecting the level,

ie, "enable IRQ5" or "enable IRQ7". Installing the

jumper may combine selecting a level and enabling

IRQ. Removing the jumper generally disables IRQ.

2. The port does not have IRQ capability

Most ports DO have interrupt capability, but a

small percentage of older systems(at a guess %1 to

%5) do not. More laptops lack interrupts than add-

in cards. Add-in cards without IRQ are rare. Most

reports of non-functional parallel port IRQs in

desktop systems can be tracked to configuration

problems which create interrupt conflicts (see #3)

which effectively disable IRQ.

3. There is an IRQ conflict with another device

It is possible that another device in your system

is preventing the parallel port IRQs from

functioning.  For example, in IBM-PCs and XTs, the

disk controller may use IRQ 5 and prevent parallel

ports configured to IRQ5 from creating interrupts.

Sound cards frequently use IRQ5 and are likely

candidates for creating such IRQ conflicts.

To isolate this problem, remove all boards from

your system which are not required to perform the

test. In other words, strip down to just the

parallel port, floppy controller and video, and

repeat the test.

If possible, configure the parallel port to IRQ7.

If the port can use IRQ7 interrupts, but not IRQ5

interrupts, then an interrupt conflict is likely.

"IRQ determined by loopback"

Parallel Port Information System                                    Page 26

---------------------------------------------------------------------------

The loopback was detected, is operating correctly,

and the port's IRQs are functioning correctly.

The IRQ level found is given in the IRQ column.

"IRQ estimated by port float"

No loopback connector was detected, but the port

is generating IRQs anyway. This probably means

that the port is connected to an inactive

(unpowered) external device. Treat IRQs determined

in this way with some skepticism; better to

disconnect the device and use a known-good

loopback connector.

"IRQ determined by EPP hardware"

If the Port Type is determined to be Intel EPP,

then Parallel does not use the IRQ test, but

rather reports the IRQ which is standard for an

Intel EPP port at that IO Address. This only

applies to actual Intel SL type EPP ports.

"IRQ noise is present"

Parallel has detected extraneous interrupts

occuring on IRQ level 5 or 7 during the IRQ test.

This condition is reported when the total number

of interrupts occurring on both IRQ5 and IRQ7

exceeds the number of interrupts Parallel expects.

This can mean several things:

1. PIC errors. The PC's interrupt controller (PIC)

is generating errors as described below.

2. An active device other than the parallel port

under test is generating interrupts on IRQ 5 or 7.

This is typically a sound card or another parallel

port with a enabled, floating, IRQ line.

3. The port's IRQ line is electrically noisy. This

has been seen with boards which have dual parallel

ports where transitions in one port's IRQ line

causes noise on the other port's IRQ line.

This result may be ignored if you are not having

any problems, but be informed that this condition

is quite likely to confuse parallel port software

which tries to autodetect the port's IRQ level.

Parallel Port Information System                                    Page 27

---------------------------------------------------------------------------

Noisy IRQ lines can also slow down or break driver

software which is not intelligent enough to defend

itself against this relatively common situation.

TECHNICAL NOTE:     PIC errors are usually found with parallel port

chips whose "open" condition on the ACK line may

allow the IRQ signal to float randomly. This

effect creates PC interrupt controller errors

which are redirected by the PIC to IRQ7 where they

can be mistaken for "real" interrupts.

An PIC error condition is defined to occur when

the PIC attempts to service the requesting device

but the device's interrupt line has gone inactive.

These are also known as "lost interrupts", which

are not really lost but rather redirected to IRQ7.

A noisy IRQ line can create the appearance of lost

interrupts and this confuse driver software.

The effect of lost interrupts being redirected to

IRQ 7 explains why some parallel port driver

software works well on IRQ7 but not IRQ5, or vice

versa. The interrupt is lost, redirected to IRQ 7,

and thus becomes found again.  Strange.

Parallel port interrupt handlers have to account

for conditions that are generally not seen by

other device drivers. If you are having problems

with parallel port software you may want to try

configuring the port to the other interrupt level.

"Data port has stuck bits!"

The rotating bit test has detected bits in the

data port which cannot be set or cleared properly.

This can mean that an enternal connection is

preventing the datalines from makeing the expected

transistions.  You may have inserted the Loopback

incorrectly.

Of course if there is no external device present,

then something else must be happening! It could be

that an active device is forcing some of the data

lines to a fixed level. It could also mean that

your port exists, but some of the data bits are

not working, which is a polite way of saying that

your port is busted.

A false Port Type may be reported in this case.

Make sure that the loopback, if used, is correct.

Make sure that all devices are disconnected.

Parallel Port Information System                                    Page 28

---------------------------------------------------------------------------

"IRQ style is INVERTED"

The port's IRQ line functions in an opposite sense

from the PC standard. A significant amount of

software will fail to work with such ports. There

is no remedy for this condition in general; the

best we can advise is to use a different port if

you have a bus system or try other CMOS settings

on a laptop if that is possible. You might try

contacting your vendor.

Parallel may falsely report inverted IRQ if there

is an external device connected to the port.

"Possible IRQ Conflict"

Two or more ports in your system are using the

same IRQ level. If you are not using interrupt-

driven parallel port peripherals or printing

software then you might simply ignore the warning

if you are not experiencing any problems. However,

you should know that some very strange results can

occur when two parallel ports are  trying to drive

the same IRQ line. Sometimes one or both ports can

falsely appear to have INVERTED IRQ.

The best solution is to move one of the ports to

another IRQ level, if that is possible.

"BIOS Data doesn't match hardware"

The contents of the BIOS LPT Table does not match

the expected initial state at boot time.

This is probably due to a parallel port device

driver, a SWAPLPT program, or print spooler having

fiddled with the BIOS data, in which case it may

be considered normal operation.

Network drivers (typically the redirector clients)

also commonly rearrange the BIOS LPT Table in

other to provide network print redirection.

This information is useful if you are having

problems with print redirection. Also, some

parallel port device drivers do not examine the

hardware directly to determine which ports exist,

but rather only look at the BIOS. If you have run

other parallel port software which has changed the

BIOS in a non-standard way, then these device

drivers may fail to find any parallel ports, or

may try to access the wrong port with peculiar

results. This can often be solved by changing the

Parallel Port Information System                                    Page 29

---------------------------------------------------------------------------

order of loading or by manually configuring the

failing driver.

"Port has latent EPP tendencies"

The port is an Intel SL EPP port, but it is

currently configured to a non-EPP mode. Use your

vendor-supplied CMOS or separate configuration

program if you need to enable the EPP capability.

Some vendors do not ship the ability to enable

EPP; an appropriate utility may be available on

the vendor's BBS.

In particular, the EPP function of Compaq SL

laptops may be enabled with their SETEPP utility.

"EPP port is Intel SL flavored"

The EPP port is an Intel SL port (360SL chipset).

It is possible for Parallel to report a false

Intel EPP detection on some systems. One Toshiba

system (3600) has an 360SL onboard which does

respond to EPP detection but is NOT connected to

the outside world - instead their custom ASIC is

used to implement the parallel port. So Parallel

will report a false EPP result on this laptop.

"EPP port is vanilla flavored"

The EPP port is NOT an Intel SL port, but does

pass a generic EPP write/readback test. This test

is only attempted if the EPP BUSY line is not

HIGH. This avoids hanging the CPU on EPP ports

which do not implement the access watchdog timer.

Parallel Port Information System                                    Page 30

---------------------------------------------------------------------------

Automatic IRQ detection

-----------------------

Parallel uses two methods for automatic detection of a port's IRQ

level: port float or the external loopback connector.

Understanding Port Float

------------------------

Many parallel ports will "float" in a unstable logic state when

connected to an unpowered external device such as a printer. This

unstable logic state may cause the port to generate a limitless

stream of apparent interrupts to the CPU if the port's interrupt

line is enabled. This peculiar behavior is used to advantage by

Parallel.

If you have a unpowered (powered off) printer, or other device,

attached to a port, then Parallel may be able to detect the

interrupt level using this floating port behavior. In this case,

Parallel will display "IRQ determined by port float". You should

treat the IRQ determined in this way as a "best guess". To be sure

of the IRQ, you need to disconnect the floating device and use

loopback.

In some cases, attaching a unpowered device to a port will enable

Parallel to detect IRQ by loopback, since the combination of port

and device appears indistinguishable from a loopback connector to

Parallel.

FLOATNOTE:  Powered on devices usually hold the interrupt line in a

stable state and prevent IRQ autodetection. Remember

that  it is best to disconnect your parallel port

device(s) and use the loopback connector when you need

the absolute best results from Parallel.

EXAMPLE:  On my home system, an unpowered HP LaserJet II printer

on an 8bit port "floats" and Parallel reliably reports

IRQ5. Attached to a 4bit port on the same system, the

LaserJet is indistinguishable from a loopback

connector.  So turning the LaserJet off converts it

into an expensive, but handy, paperclip. Turning it on

prevents IRQ detection.

TECH NOTE:  Whether or not a given device and port combination will

float, emulate loopback, or be invisible to Parallel is

determined by the pull-ups on each end, CMOS vs TTL

logic behavior, and the power available at the port to

a unpowered device. In the above example, I suspect

that enough power is provided by the 8bit TTL port to

drive the unpowered CMOS logic in the LaserJet into an

unstable oscillating state, which appears to the system

as a frenzy of level 5 IRQs.

Port float is a handy effect, but use Loopback for best results.

Parallel Port Information System                                    Page 31

---------------------------------------------------------------------------

The most reliable way to determine IRQ is to use an external

loopback connection. Loopback is described in the next section.

IRQ detection using the LoopBack Feature

----------------------------------------

******************

* IMPORTANT NOTE *

******************

First, DECIDE NOT TO SCREW UP YOUR PORT by crudely jamming wires

into the wrong places with too much force.  REMEMBER that the

female DB-25 connector is likely to be made out of CHEAP, BREAKABLE

PLASTIC, so decide right now not to break it. WE ARE NOT LIABLE FOR

ANY DAMAGE YOU MAY DO FOR WHATEVER REASON.

The danger is that you will push too hard when inserting the

loopback wire and force the female connector pin out of its

position in the DB-25 shell. That is why WE SUGGEST MAKING A

LOOPBACK PLUG using a male DB-25 connector as described below. If

you cannot make a loopback plug, then use a parallel port extension

cable instead. This protects your port from physical damage and

also makes it a lot easier to work with.

The port is supposed to be electrically designed to tolerate the

loopback perfectly well. However there is a DANGER OF DAMAGE BY

STATIC DISCHARGE so make sure that you GROUND YOURSELF TO THE PORT

before inserting the loopback wire. It would be terrible if you

blew out a $25 parallel port on a $1000 Pentium motherboard simply

because you failed to PAY ATTENTION TO THESE PRECAUTIONS.

PROCEED AT YOUR OWN RISK.

The other danger is that you will apply the loopback to a port

which is not really a parallel port at all. DON'T DO THIS. If you

are not sure of what you are doing STOP RIGHT NOW. We have no

control over what you might damage if you try this with a port

other than a parallel port.

Whew... now on with the instructions.

1. Find the parallel port(s) you want to test. They will be

female DB-25 connectors. MAKE SURE THAT YOU HAVE CORRECTLY

IDENTIFIED THE PARALLEL PORT. Disconnect any devices

(printers, network adapters, external disks, etc) which might

be attached to other parallel ports on your system.

2. Make sure that the connector IS NOT SOME OTHER TYPE OF

CONNECTOR like an external SCSI connector or a COM port.

SCSI ports can be either female DB-25s or a larger connector,

like a 37, 40, or 50-pin connector. COM ports are usually

Parallel Port Information System                                    Page 32

---------------------------------------------------------------------------

male DB-25 or small male DB-9 connectors, but this is not

guaranteed.

Look in your manual if there is any doubt. DON'T PROCEED

unless you are sure you have identified the parallel port(s)

correctly.

3. Connect the Loopback Plug and go to Step 7.

If you do not have a Loopback Plug, use a straight thru M-F

parallel port extension cable to prevent damage to your port.

Connect the male end of the extension cable to your port and

place the loopback wire into the female end of the cable.

|---

expensive    |   | +-+     cheap     +-+

PC        |   | | |---------------| | <- loopback here

parallel port  |   | +-+     cable     +-+

|---

4. Identify Pin 10 and Pin 9 on the female end of the extension

cable. Refer to the illustration below. Notice that a DB-25

connector is wider on one side.  The pins you want are the

4th and 5th from the left on the wider edge.

USE A FLASHLIGHT TO FIND THE MARKINGS IF YOU HAVE ANY DOUBT.

Front View         -------- Pin 10 - 4th from left on wider edge

Female DB-25       |   ---- Pin  9 - 5th from left on wider edge

Connector          |   |

v   v

____________________________________________________________

/                                                            \

\     13  12  11  10   9   8   7   6   5   4   3   2   1     /

\                                                          /

\     25  24  23  22  21  20  19  18  17  16  15  14     /

\______________________________________________________/

Pin 10 is the interrupt line (the ACK line)

Pin  9 is the high order data line (the 80h bit)

4. Create a loopback wire. A standard paperclip works fine. Just

straighten it out and bend it into a "U" shape with the ends

1/8 inch apart. Make sure that the paperclip is SMOOTH and

has NO ROUGH EDGES that might damage the plastic connector.

5. GROUND YOURSELF to the PC and to the parallel port by

touching your hand and the loopback wire first to the PC's

metal chassis near the parallel port connector and then to

one of the connector nuts or the connector metal housing

itself. Do this to DISCHARGE ANY STATIC ELECTRICITY you may

have about you (created for example by scooting across the

rug in a silk negligee on the way to your PC or by stroking

your cat with woolen gloves while purusing this document).

Parallel Port Information System                                    Page 33

---------------------------------------------------------------------------

Parallel ports are usually resistant to static discharge, but

not all ports are created equally, so don't ask for trouble.

6. Carefully insert the paperclip into the extension cable's

female DB-25 connector so that Pin 9 is connected to Pin 10.

DON'T FORCE IT. It will insert at most about 1/4 inch (about

.5 cm) and there should be absolutely no physical resistance.

7. Now run Parallel. It will automatically use the loopback to

determine which IRQ level the port is actually using. You may

loopback more than one, or all, ports at the same time.

Parallel will report that is has detected a proper loopback.

8. Remove the loopback when you are done.

9. Congratulate yourself on using reasonable handling

precautions when working with expensive equipment.

TechNote: Making Your Own Loopback Connector

--------------------------------------------

We strongly advise that you make a loopback connector, inside of

using the expedient paperclip approach.

Simply obtain a male DB-25 connector and solder a permanent

connection bewteen Pin 9 to Pin 10 on the back of the connector.

Male DB-25 connectors are inexpensive and available in electronics

parts stores. Cheap DB-25 jumper boxes cost only a few dollars.

This will prevent any physical damage to your port, which is the

major source of risk with this procedure, and the reason for our

incessant nagging about precautions. This sort of loopback

connector will be no more dangerous to your system than a printer,

and is a handy accessory if you frequently use Parallel.

Connect Pin 10 to Pin 9

Back View          -----    Pin 10 - 4th from left on wider edge

Male DB-25         |   |    Pin  9 - 5th from left on wider edge

Connector          |   |

v   v

____________________________________________________________

/                                                            \

\     13  12  11  10   9   8   7   6   5   4   3   2   1     /

\                                                          /

\     25  24  23  22  21  20  19  18  17  16  15  14     /

\______________________________________________________/

Pin 10 is the interrupt line (the ACK line)

Pin  9 is the high order data line (the 80h bit)

Parallel Port Information System                                    Page 34

---------------------------------------------------------------------------

Miscellaneous Topics

--------------------

Technote: Pseudo-Bidirectional Ports, A Special Case

----------------------------------------------------

A 4bit port is pseudo-bidirectional (also called pseudo-8bit,

sometimes semi-8bit) when it is capable of performing 8BIT input

when connected to a device which has strong drivers. So this

pseudo-bidirectionality comes about as the relationship between the

port and the device it is connected to.

This type of port will appear to be a 4BIT port with most parallel

port external peripherals, since low power CMOS drivers are by far

the most common in these external devices.

A few peripherals have stronger TTL drivers which can achieve 8BIT

operation by overdriving the port. There are even sub-classes of

ports which can achieve the effect depending on what data is used.

Some vendors describe their ports as bidirectional when actually

they are only pseudo-bidirectional and will function as 4BIT ports

with most peripherals. True 8BIT ports are preferable because they

have a significant speed advantage with most external peripherals.

Indeed, some parallel port peripherals require true 8BIT behavior.

Identifying pseudo-bidirectional ports

--------------------------------------

Step 1  =>  Read the cautions and disclaimers applying to loopback

Step 2  =>  Read them again

Step 3  =>  Select the port. The test only applies to 4BIT ports.

Step 4  =>  Attach a parallel port extension cable to the port

Step 5  =>  At the female DB-25, loopback Pin 8 to Pin 21 (GND).

Step 6  =>  Run Parallel

Step 7  =>  If Parallel reports "Data port has stuck bits" then the

port is likely to have semi-bidirectional capability.

Parallel Port Information System                                    Page 35

---------------------------------------------------------------------------

Parallel Port Standards

-----------------------

4bit and 8bit parallel ports

----------------------------

There is no true standard for these types of ports other than

the actual behavior of existing systems.

The original IBM PC Technical Reference manual gave a schematic

for the parallel port which was ambiguous about the ability to

input from the printer port.

Various vendors interpreted the ambiguity differently ... and

this appears to have been the genesis of a persistantly loose

and confused industry attitude toward parallel port standards.

EPP Specification

-----------------

There are two substantially incompatible standards for EPP

parallel ports: the original EPP Standards Committee version 1.7

standard, and the IEEE 1284 Committee standard.

We call these two types of EPP ports "EPP 1.7" and "EPP 1284".

The two standards are sufficiently similar that new peripherals

may be designed in such a way as to support both standards, but

existing EPP 1.7 peripherals will generally not operate with EPP

1284 ports.

The reference implementation of EPP 1.7 is the Intel 360SL chip.

There are several implementations of EPP 1284 including the SMC

66x, the National PC87xxx family, and perhaps others. Chipset

vendors are rapidly adding advanced parallel port functionality

to their product lines.  We will continue to review these chips

as they appear from other manufacturers.

ECP Specification

-----------------

The ECP port standard was developed by Microsoft and Hewlett-

Packard. The standard specifies a PC parallel port with certain

I/O registers and features and an external signalling behavior.

The external signalling specification has been incorporated into

the IEEE 1284 specification as the ECP mode of 1284.

Reference implementations of ECP include the SMC 66x, the

National PC87xxx, and others. These chips vary in the processes

required to configure, and specific PC implementations will vary

in their setup and/or BIOS support.

Parallel Port Information System                                    Page 36

---------------------------------------------------------------------------

IEEE 1284 Specification

-----------------------

The IEEE 1284 specification defines the electrical signalling

behavior external to the PC for a multimodal parallel port which

may support 4bit, 8bit, EPP, and ECP modes of operation. Not all

modes are required by the 1284 specification and the standard

allows makes some provision for additional modes.

The IEEE 1284 standard is targeted at specifying the behavior

between a PC and an attached device, most specifically attached

printers, although the specification is of interest to vendors

of parallel port peripherals (disks, LAN adapters, etc, etc).

The IEEE 1284 standard is currently maintained by Larry Stein of

Farpoint Communications (see Vendor Contact section).

The IEEE standard has additional "substandards", including IEEE

1284.3 (daisychain and multiplex standard) and the committee is

beginning work on operating system standards and certification

processes. The IEEE 1284 style parallel port is required for PCs

to comply with the PC95 platform specification. We know of no

parallel port implementations with native plug-and-play support.

Various non-PC/DOS/Windows hardware and software vendors (Sun,

Apple, Next) have shown interest in the committee's efforts. It

is very likely that future versions of Windows will support the

1284 specification, at least in the area of high-speed printing.

The 1284 specification will also likely be approved as a

component of the Desktop Management Task Force (DTMF) standard

for PC resource management.

The downside of the IEEE 1284 specification is that it does not

specify the PC's internal interface to a 1284 compliant parallel

port, either at the ISA register level or a software API level.

Each chipset vendor is implementing subtly different behaviors

in their chipsets, especially as regards configuration and setup

- a process which is recreating the same system and software

management mess that exists on earlier generations of parallel

port technology. In order words, the IEEE 1284 specification

does not answer the basuc question "What is a parallel port and

how do I perform I/O to it?".

Perhaps the Committee's efforts will successfully expand to

provide a unified solution, and perhaps such efforts will again

be too little, too late in the technology curve to achieve a

unified practical standard.

To date, the IEEE 1284 standard does not represent a unified

solution to the challenges and potential of future parallel port

technology.  It represents a tolerable, if uninspired, next step

in the evolution of particular classes of the technology.

Parallel Port Information System                                    Page 37

---------------------------------------------------------------------------

Parallel Port Standards Diagram

-------------------------------

The following diagram attempts to depict the interrelatedness of

various standards affecting the PC parallel port.  It is useful

to keep in mind the distinctions between systems, interfaces,

port designs, and specific chips when considering parallel port

issues.  Since the parallel port has been a "poor relation" in

the evolution of these standards, there are gaping holes in the

overall standarization process.

For example, there is no standard for hardware independent I/O

to any parallel port and device.  Existing proposed standards

are inadequate to the purpose.  Such a standard must provide a

level of adaptive flexibility in expanding dynamically to

incorporate new instances of ports, devices, and I/O methods.

Actually, the Plug-and-Play standards concept is an interesting

paradigm for such a parallel port software standard, which would

treat ports, devices, and I/O methods as pluggable entities in a

comprehensive and real-time extensible software architecture.

-------------------------------------------------------------------

|

Platform  | PC95 Platform       DTMF            Individual

Level     |   Standard        Standard        Operating System

Standards |                                  Software Standards

|                                                 ^

------------------------------------------------------------|------

|                                                 |

Interface |   IEEE 1284 Interface     possible future IEEE  |

Standards |         Standard          software standards  --- ?

|

-------------------------------------------------------------------

|

PC Port   |   EPP 1.7 Port            MS/HP ECP Port

Standards |    Standard                  Standard

|

-------------------------------------------------------------------

|                                           C&T

Specific  |    Intel        SMC        National       Cirrus Logic

Chips     |    360SL        66x        PC87xxx        StarTech

|                                           others

|

-------------------------------------------------------------------

|

Example   |   SL laptops    Intel      IBM            unknown

Specific  |   with 360SL    Pentium    ThinkPad       or

Systems   |   or equiv.     Mthrbrd    755            forthcoming

|

-------------------------------------------------------------------

Parallel Port Information System                                    Page 38

---------------------------------------------------------------------------

Parallel Port Buyer's Guide

---------------------------

This Guide is intended to serve as a starting point for user's in

search of quality parallel ports and peripheral devices.

Parallel Technologies makes no warranties regarding these products

and indeed we remind you to independently verify that the product

will perform as expected in your environment.

Parallel's evaluation and recommendation policy is:

1. We recommend only those products that we ourselves have

tested extensively.

2. We make only positive recommendations. Inferior products will

generally NOT be mentioned unless we consider them patently

offensive to the user.

3. Vendors are invited to arrange evaluations of their products

so that they may be included in the Buyer's Guide. We will

evaluate the product and propose a short description of it's

capabilities, features, and quality. The vendor has the

option of requesting that the product not be listed.

4. We are planning to expand the coverage in the Buyer's Guide

to include parallel port peripheral devices: LAN adapters,

disks, SCSI adapters, CDROMs, and so forth. Again, interested

vendors are welcome to arrange and actively contribute to a

review of their product. The "no-pan" rule will apply.

Direct Sales suppliers of products are listed in those cases where

such suppliers are known and have proven reliable in the past.

See the Vendor Contact section for specific vendor information,

including telelphone numbers and email addresses, if applicable.

4bit parallel ports

-------------------

This type of port is not recommended. Although most external

parallel devices will support this type of port quite well, it

is considerably slower at input operations than all other port

types and has no cost advantages.  At a minimum, get an 8bit

port or better yet, one of the advanced port types (EPP, ECP, or

a multimodal port which support both EPP and ECP).

8bit parallel ports

-------------------

8bit parallel ports are adequate for CURRENT printing purposes

and may be used with all parallel port devices.  However, you

Parallel Port Information System                                    Page 39

---------------------------------------------------------------------------

should consider buying an advanced parallel port (particularly

an EPP/ECP capable multimodal port) when buying new systems.

Mfg & Model     Features                        Suppliers

--------------  -----------------------------   ---------------

STB DSP 550     basic 2S+P ISA add-in card      Essential Data

requires jumper for 8bit mode   JAZ COmputer

has 16550 serial ports

--------------  -----------------------------   ---------------

Everex MagicIO  very clean signals

hard to find

--------------  -----------------------------   ---------------

other           8bit generic 2S+P I/O cards

Parallel reliably detects them

highly variable quality

--------------  -----------------------------   ---------------

QuickPath       multifunction ISA add-in card   Essential Data

PortFolio       IDE, 4 floppy, game port

4 serial ports with 16650's

2 parallel ports

EPP Parallel Ports

------------------

EPP-capable parallel ports are generally found only on laptops,

as a result of their introduction on the Intel 360SL I/O chip

which was part of the SL chipset designed for laptops. We have

listed one supplier (FarPoint) for a ISA add-in card suitable

for desktops.

Be sure that you identify the VERSION of EPP support the port

provides, EPP 1.7 or IEEE 1284 EPP.

EPP 1.7 is the older Intel-style EPP which is

compatible with many existing parallel port devices

such a Xircom LAN adapters, Trantor SCSI adapters,

SyDOS cartridge disks, and so forth.

IEEE 1284 EPP is a new specification for EPP which is

not completely backward-compatible with the EPP 1.7.

This means that most existing parallel devices do not

work with IEEE 1284 EPP.

Tech Note:  It is possible to design parallel port devices so that

they are compatible with BOTH the EPP 1.7 and 1284 EPP

specs, however existing devices typically don't do this

yet. Existing devices generally support only EPP 1.7.

Parallel Port Information System                                    Page 40

---------------------------------------------------------------------------

If you are buying a parallel port device (LAN, Disk,

CDROM, etc, etc) for general use, consider holding out

for a version that support BOTH EPP 1.7 and IEEE 1284

EPP or ECP modes of operation.  This gives you support

for the large installed base of SL style laptops and

compatibility with future system where IEEE 1284 style

ports will predominate.

Mfg & Model              Features                 Supplier

--------------  -----------------------------   ---------------

Intel SL-based  any laptop fully implementing   many and varied

laptops         Intel 360SL compatibility

or compatibles  more than 50 models available

provides EPP 1.7 capability

--------------  -----------------------------   ---------------

Farpoint Fport  ISA add-in card with EPP 1.7    Farpoint

downloadable personalities

has extra useful IO addresses

has extra useful IRQ levels

good documentation

value-added software for

port configuration and drivers

for fast DOS / NOVELL printing

excellent tech support

When buying an EPP-capable laptop for use with EPP external

peripherals, it is important to BE VERY CLEAR about what you are

getting.  Even different models from the same manufacturer will

have different implementations of EPP.  CHECK COMPATIBILITY with

the specific peripherals you intend to use.

Example 1:  Various SL-based Compaqs (SL25s, Conturas) have long

has excellent implementations of EPP 1.7. However the more

recent Compaq Aero appears to have a unique implementation which

is not fully SL compatible.

Example 2:  The IBM Thinkpad 750 has an SL-compatible EPP 1.7

port, but the 755 model has a non-SL-compatible EPP port.

Devices which work on one 700 model do not work on another.

NOTE: This is not an indictment of these laptops by any means,

as they as fine systems overall. It IS A REMINDER that you must

independently verify the current state of compatibility between

any particular EPP-capable laptop and parallel devices you plan

to use with them. Laptop vendors generally will not know the

answers to your questions about parallel ports and the best bet

is the peripheral vendor.

EPP 1.7 type parallel ports MAY OR MAY NOT be supported by any

specific new IEEE 1284 type peripheral. We predict that most

developers of new devices will want to support EPP 1.7 in order

to be compatible with the install base of SLs, however they may

Parallel Port Information System                                    Page 41

---------------------------------------------------------------------------

opt to just support IEEE 1284 style EPP which leaves the SLs up

the creek. It is possible to support both.

So there is a certain risk factor in buying a laptop with only

EPP 1.7 support today ... as an increasing number of systems are

appearing with multimodal 1284 style ports. On the other hand,

buying a laptop without EPP 1.7 leaves you incompatible with a

large variety of existing external peripherals - you will have a

bit of a wait as peripheral vendors scurry to support IEEE 1284.

In the best of worlds, you will buy a system which supports all

the IEEE 1284 modes, EPP 1.7 to boot, and comes with a handy

utility which allows you to configure your port easily.

ECP Parallel Ports

------------------

There are some very nice chipsets coming out for multimodal

parallel ports which support the multiple parallel port modes.

These chipsets have been appearing in systems since early this

year and have just now reached the stage where they are safe

consumer choices given a little caution.

In some cases, early "steps" of particular chips had significant

flaws and the buyer should verify that he is getting the latest

step - not something that's been in a warehouse since Easter.

Vendors may or may not provide user level support for selecting

the parallel port mode. Check that any PC you are considering

has appropriate CMOS port configuration or at least a separate

utility for the purpose. Be aware that vendors may have BIOS

upgrades which retroactively enable such support.

If you need immediate support for existing parallel port devices

operating in EPP mode you should INDEPENDENTLY VERIFY that the

device works with the particular system you are considering. Be

aware that existing EPP 1.7 type devices may or may not work

with any particular new multimodal parallel port. It is still

early days yet for ECP support in parallel port devices and this

caution also applies to any early versions of such devices you

might consider - check that they work with your specific system.

Mfgr & Model    Features                        Supplier

--------------  -----------------------------   ---------------

Intel Premiere  motherboard with SMC 665 chip   used in many

Pentium PCI     66xQPF chip has various bugs    Pentium systems

Motherboard     be sure you get the 665GT chip  from various

requires Plato version 10 BIOS  "manufacturers"

to enable EPP/ECP in Setup

Parallel Port Information System                                    Page 42

---------------------------------------------------------------------------

--------------  -----------------------------   ---------------

Other systems   be sure you get the 665GT chip  various

with SMC 66x    check for software support

--------------  -----------------------------   ---------------

SIIG IDE        multifunction ISA add-in card   superstores, etc

Professional    IDE, floppy, 2serial, parallel

uses the SMC 66x chip

again, avoid the QPF version

no port configuration software

no value-added software

decent jumper documentation

--------------  -----------------------------   ---------------

Farpoint        ISA add-in card with SMC 66x    Farpoint

Fport Plus      serial and parallel port only

good configuration utility

fast DOS / Windows driver

1 16650 serial port

excellent tech support

--------------  -----------------------------   ---------------

Farpoint        ISA add-in card with SMC 66x    Farpoint

Fport IDE       IDE, 2 serial, parallel

good configuration utility

fast DOS / Windows driver

2 16650 serial w/ extra IRQs

excellent tech support

--------------  -----------------------------   ---------------

Kouwell         typical clone SMC 66x ISA card  Essential Data

IDE, 2 16550 COM, parallel,

no value added software

very low cost

check that you get the GT step

Advanced Parallel Port Chipsets

-------------------------------

Mfg & Model     Features                        Supplier

--------------  -----------------------------   ---------------

SMC 66xQPF      multifunction I/O chip          SMC

multiple parallel port modes

IEEE 1284 EPP and ECP

no EPP 1.7

has bad serial port FIFO bug

EPP mode has minor bugs

good chip, but use the GT step

--------------  -----------------------------   ---------------

SMC 66xGT       corrected version of 66xQPF     SMC

IEEE 1284 EPP and ECP

adds EPP 1.7

fixes various bugs, com FIFO

Parallel Port Information System                                    Page 43

---------------------------------------------------------------------------

--------------  -----------------------------   ---------------

National        PC87xxx SuperI/O chip family    National

Semiconductor   322 is EPP 1.7 only

332, 334, 303, 323, and 306

have EPP 1.7, EPP 1284 & ECP

various features on chips:

IDE, 16650 serial, floppy,

keyboard, clock, serial IR

5 volt and 3.3 volt versions

--------------  -----------------------------   ---------------

Chips &         information not available       C&T

Technologies    at this time, watch this space

Parallel Port Information System                                    Page 44

---------------------------------------------------------------------------

Vendor Contact Information

--------------------------

Parallel Technologies     Jay Lowe, VP          206-869-1136 voice

parallel port wizards   CIS 76640,203         206-869-1133 fax

contract development    Don Schuman, Pres

product licensing       CIS 72242,3631

DirectConnect(TM)

technology licensing

FarPoint Communications   Larry Stein           805-726-4420

direct sales vendor     CIS 71514,1057

specialize in parallel  IEEE 1284 chairman

very experienced

Essential Data            Bruce McKeag          800-795-4576 voice

direct board sales      CIS 70572,2670        408-955-0440 voice

active on Compuserve                          408-955-0821 fax

JAZ Computer Products     J. Zarifis            914-632-9531 voice

direct board sales      CIS 71042,1251

GEnie J.ZARIFIS

Standard Microsystems     Robert Gross

chipset vendor

National Semiconductor    Kishor Padmanabhan    408-721-4563

chipset vendor

Chips & Technologies      forthcoming

chipset vendor

IEEE 1284 Committee       Larry Stein           805-725-4420

CIS 71514,1057

- also -

EET:STDFRM CIS forum

Reputable vendors of parallel port technology are invited to

forward us a short listing of their interests

and contact information for inclusion here.

Parallel Port Information System                                    Page 45

---------------------------------------------------------------------------

Technical Support

-----------------

Parallel is Freeware, and comes with Freeware-style support. It has

been extensively tested with a range of systems from original PCs

to Pentiums and works with all parallel port devices we've seen.

We are willing to offer friendly assistance on an as-available

basis, but only via Compuserve mail (CIS 76640,203), via Internet

Email to jaylowe@halcyon.com. or in the Compuserve IBMHW forum.

Alas, we do not offer free telephone assistance to the public.

If you are in dire need of technical support in solving port

related problems, you may open a support account by contacting:

Mailing Address                         Shipping Address

---------------------------             ---------------------------

Parallel Technologies, Inc.             Parallel Technologies, Inc.

PO Box 7                                10603 170th CT NE

Redmond, Wa 98073-0007                  Redmond, Wa 98052

(206) 869-1136

(206) 869-1133 (fax)                    ( for UPS, FEDEX, etc )

Compuserve: 76640,203

InterNet:   76640.203@COMPUSERVE.COM

-or-  :   jaylowe@halcyon.com

Custom versions of the Parallel Port Information Utility, and

source code, are available.

Vendors of parallel port hardware are invited and requested to

contact us so that we may ensure that Parallel is compatible with

their products.

Permission is given to distribute and use Parallel without charge,

provided you adhere to the terms and conditions of the license

agreement given below. Specifically, you may not charge anyone for

for sharing or using Parallel, and you may not modify it or change

it. Nor may you use it as part of a commercial product or service

without our written permission.

Vendors may freely distribute Parallel on an individual basis,

without charge, by electronic BBS or other means.  However, you may

not include it in or with your product without our written

permission.

We welcome your comments and suggestions by Email, phone, or fax.

If you have particular features you would like to see supported by

Parallel, we'd be more than glad to hear from you.

Parallel Port Information System                                    Page 46

---------------------------------------------------------------------------

About Parallel Technologies, Inc.

---------------------------------

We at Parallel Technologies' have been creating innovative parallel

port devices and software since 1989. Here are a few highlights:

Firsts, Bests, and Mosts

------------------------

o  1st ... parallel port network adapter

o  1st ... enhanced, bus-speed, parallel port

o  1st ... parallel port external SCSI disk

o  1st ... passthrough printing capability

o  1st ... automatic self-configuring drivers

o  1st ... universal support for all types of parallel ports

o  1st ... network adapters with multiple media support

o  1st ... Windows 3.1 multimedia driver for parallel adapter

o  1st ... Multiple Parallel Device API for shared devices

o  1st ... intelligent parasitic power for self-powered devices

o  1st ... demonstration of daisychained parallel peripherals

o  1st ... demonstration of multiplexed parallel peripherals

o  1st ... Parallel Port Expansion System (now IEEE Multiport)

o  1st ... Parallel Port Developer's Kits

o  1st ... general purpose parallel port networking (DirectNet)

o  1st ... Win 95 parallel port networking (DirectNet VCOMM)

o  PC MAG Editor's Choice for Ethernet parallel port NIC

o  most total network adapters  --  7 and counting...

o  most total parallel port peripherals  --  25 and counting...

Partial List of Parallel Technologies' Developments

---------------------------------------------------

o  parallel port Arcnet adapter

o  parallel port Ethernet Adapter (two versions)

o  parallel port Token Ring Adapter (two versions)

o  parallel port portable SCSI disk drive

o  parallel port portable IDE disk drive

o  parallel port MIDI interface for Windows

o  parallel port Zero-Slot mouse adapter

o  parallel port IR Wireless Adapter

o  parallel port DSP signal processing interface

o  parallel port Multiport daisychaining system

o  parallel port Multiport 4-way parallel device multiplexor

o  DirectNet ODI driver for parallel port networking

o  DirectNet VCOMM driver for parallel port networking

o  DirectNet 4BIT cables for parallel port networking

o  DirectNet ECP cables for parallel port networking

o  DirectNet Universal cables for parallel port networking

Parallel Port Information System                                    Page 47

---------------------------------------------------------------------------

o  DirectNet 4BIT/ECP and UCM cable interface specification

licensed to Microsoft for joint Microsoft/PTI publication

o  Parallel Port Developer's Kit (Level 1)

o  Parallel Port Developer's Kit (Level 2)

o  Super IEEE 1284 Printing System Developer's Kit

o  DirectNet Parallel Port Networking Manufacturing License Kit

o  DirectNet Parallel Port Networking Software License Kit

Many of these products, or their hardware and software components,

are available for OEM licensing.

We have in-depth experience with the wonderful and sometimes

bizarre world of the PC parallel port and have served on both the

EPP and IEEE 1284 standards committees. Having created more than

TWO DOZEN parallel port devices with their associated drivers and

software utilities, we are the most experienced source for off-the-

shelf technology, developer tools, and contract services.

Here are a few of our current and forthcoming products that you may

find interesting.

Parallel Port Developer's Kits

------------------------------

Parallel Technologies offers parallel port Developer's Kits which

contain those hardware and software components necessary to build

very high-speed, consumer-quality external devices.

The Developer Kit technology has been used in over 25 parallel port

devices including network adapters, parallel port SCSI and IDE

disks, parallel port wireless IR adapters, parallel port MIDI and

mouse interfaces, parallel to serial interfaces, and so forth.

The software includes support for all types of ports, IO code that

is extremely resistant to port idiosyncrasies, and full auto-

detection of all port hardware features. The code is very well

documented. A selection of sample drivers for parallel port devices

is also available.

Level One Developer's Kit

-------------------------

Hardware schematics, timing diagrams, access methods

and all other hardware information required to

implement basic peripheral devices that are compatible

with 4BIT, 8BIT, semi-8BIT,and EPP parallel ports

The software section includes sample source code for

register and block data access to the sample device.

Parallel Port Information System                                    Page 48

---------------------------------------------------------------------------

Level Two Developer's Kit

-------------------------

Includes the Level One Kit PLUS

additional very high speed IO techniques,

passthrough port design techniques

daisychain and multiplexor Multiporting design

additional ECP hardware design techniques

additional source code for ECP I/O techniques

full source code for Parallel autodetect utility

Also includes full EPP and IEEE 1284 specifications.

Super IEEE 1284 Developer's Kit

-------------------------------

Prototyping board, printer side interface FPGA

equations, assembler source for 1284 negotiation,

Windows 3.1 driver source.

Allows the rapid development of IEEE 1284 printers.

Also Super 1284 goes BEYOND the IEEE 1284 spec to allow

very high-speed bi-directional I/O on 4BIT, semi-8BIT,

and 8BIT ports.  This means that Super 1284 printers

can operate bi-directionally at very high speeds

(upwards of 300Kbyte/sec) even on the installed base of

"old-fashioned" standard parallel ports.

Super 1284 maintains complete compatibility with IEEE

1284 ports and printers. It is a compatible enhancement

that allows printer vendors to support high-speed

operation without requiring EPP and ECP ports.

DirectNet -- The Parallel Port Networking System

------------------------------------------------

The DirectNet parallel port networking system has been

incorporated into Windows 95 and may be licensed for

inclusion in other operating environments.

The DirectNet system is a family of completely external

connection devices and network driver software that

allows PC-to-PC plug-and-play cable connections to be

used as an alternative to more expensive LAN hardware.

The DirectNet hardware technology includes 3 point-to-

point cables, and multiport hub, and a distributed

multdrop system which uses RJ-11 standard phone wiring.

Parallel Port Information System                                    Page 49

---------------------------------------------------------------------------

DirectNet supports speeds of 40 - 500+ KBytes/sec

depending on the parallel ports involved and various

other factors. The system is completely self-adaptive.

Because DirectNet provides a suite of standard network

drivers, it may be used with many existing networking

and connectivity systems such as Windows 95, Windows

for Workgroups, Netware Lite and Personal Netware, etc.

DirectNet connections have been tested with more than a

dozen commonly available networking and connectivity

software packages.

DirectNet is now in a  BETA RELEASE status and Parallel

Technologies is providing manufacturing and software

licenses vendors who wish to include DirectNet products

or capability in their product lines.

Parallel -- the Parallel Port Information Utility

-------------------------------------------------

A handy utility which examines your system's parallel

ports and reports the Port Type, IO address, IRQ level,

BIOS name, and an assortment of informative notes and

warnings in a compact and easy to read display. The

output may be redirected to a file for tech support

purposes. Parallel uses very sophisticated techniques

for port and IRQ detection and is aware of a broad

range of quirky port features.

Parallel is the definitive parallel Tech Support tool.

Parallel is Freeware, subject to a restricted license

agreement. It may be used free of charge for any

personal, non-commercial purpose. Parallel may not be

sold for profit, nor may it be distributed with any

other product sold for profit. Vendors should contact

us regarding commercial licenses.

OEM Products

------------

Most of our products are available for OEM licensing,

including network adapters, EPP-capable hard disks,

parallel-SCSI interface, parallel MIDI adapter, and the

MultiPort parallel port expansion system.

Also available are numerous off-the-shelf hardware and

software components including: device schematics, FPGA

equations, DOS and Windows driver source code, etc.

Parallel Port Information System                                    Page 50

---------------------------------------------------------------------------

Development Services

--------------------

We specialize in contract development of parallel port

peripherals and associated software including DOS and

Windows device drivers. Our broad and deep experience

with parallel port devices and our commitment to

specialization allows us to design, implement, test,

and support them very efficiently.

You might be interested in our 45/90 development offer.

We propose to convert almost any ISA peripheral into a

parallel port device in only 45 days to prototype, 90

days to production.

We also offer more limited development support, such as

developing additional device drivers or enhancements

for existing peripherals.

We are quite experienced with EPP and ECP ports and are

ready to upgrade your existing parallel port devices

for peak performance.

Consulting and Training

-----------------------

In support of in-house development, you will find our

consulting and training services invaluable in avoiding

the pitfalls of parallel port development - and achieve

the highest possible product performance and quality.

We conduct in-house trainings and design consultations

in hardware, software, and product design / management.

You may find these  trainings useful in bringing your

development and management team up to speed on parallel

port issues.

Parallel Port Information System                                    Page 51

---------------------------------------------------------------------------

Software License Agreement

--------------------------

IMPORTANT

Read this agreement before installing or using the Software. Installing

and/or using the Software on your computer indicates your acceptance of

these terms. If you do not agree to these terms and conditions, delete

the Software and all it's associated files from your computer. This

Software License Agreement gives you ONLY RESTRICTED RIGHTS to use the

Software (and all it's associated files and documentation) as follows:

YOU MAY

1.  Use the Software without charge on as many systems as you wish,

2.  Share the Software with others for their personal use, but only

if you share it intact, without any changes or modifications of

any kind, and complete with all it's documentation and associated

files. You must pass it on exactly in the same form as we release

it, or not pass it on at all.

3.  Make backup copies of the Software, so long as you place our

copyright notice on the backup copies,

4.  Request and receive limited technical support as provided in the

Freeware Product Support Policy below.

5.  Vendors may distribute the Software free of charge by electronic

or other means for the purpose of technical support, but may not

include the Software in products without our written permission.

YOU MAY NOT

1.  Use the Software in any way except as provided in this agreement,

2.  Modify, alter, copy, transfer, or reproduce the Software in any

way, except as expressly allowed in this agreement,

3.  Translate, de-assemble, de-compile, reverse program, or reverse

engineer the Software or any of its components,

4.  Charge any kind of fee for distributing the Software without our

written permission,

5.  Use the Software for commercial purposes without our permission,

6.  Sublicense, lease, rent, sell, distribute, or otherwise dispose

of the Software without our written permission.

This license is effective from the date of first use of the Software and

shall remain in force until terminated. You may terminate this agreement

by destroying any and all copies of the Software and its documentation.

Parallel Port Information System                                    Page 52

---------------------------------------------------------------------------

Parallel Technologies, Inc. may terminate this agreement if you fail to

comply with any of the terms and conditions of it. You agree to destroy

any and all copies of the Software and its documentation, in whatever

form, if we terminate this license agreement, and you agree to provide

us written certification of such destruction upon our request.

Statement of Exclusion of Warranty

ALL PRODUCTS DELIVERED PURSUANT TO THIS AGREEMENT ARE DELIVERED ON AN

"AS-IS" BASIS AND PARALLEL TECHNOLOGIES, ITS DEALERS AND DISTRIBUTORS,

EXPRESSLY DISCLAIM ANY AND ALL WARRANTIES OR CONDITIONS OF MERCHANTABLE

QUALITY OR FITNESS FOR A PARTICULAR PURPOSE.

Limitation of Liability

PARALLEL TECHNOLOGIES DISCLAIMS ALL WARRANTIES RELATING TO THIS

SOFTWARE, WHETHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE, AND ALL SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY

DISCLAIMED.  NEITHER PARALLEL TECHNOLOGIES NOR ANYONE ELSE WHO HAS

BEEN INVOLVED IN THE CREATION, PRODUCTION, OR DELIVERY OF THIS

SOFTWARE SHALL BE LIABLE FOR ANY INDIRECT, CONSEQUENTIAL, SPECIAL,

PUNITIVE, OR INCIDENTAL DAMAGES OR DAMAGES DUE TO LOST PROFITS, DATA,

OR INFORMATION ARISING OUT OF THE USE OR INABILITY TO USE SUCH

SOFTWARE EVEN IF PARALLEL TECHNOLOGIES HAS BEEN ADVISED OF THE

POSSIBILITY OF SUCH DAMAGES OR CLAIMS.  IN NO EVENT SHALL PARALLEL

TECHNOLOGIES' LIABILITY FOR ANY DAMAGES EVER EXCEED THE PRICE PAID FOR

THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM OF CLAIM.  THE

PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND

PERFORMANCE OF THE SOFTWARE.  This warranty gives you specific rights

and you may have other rights.

This agreement shall be governed by the laws of the State of

Washington and shall inure to the benefit of Parallel Technologies,

Inc. and any successors, administrators, heirs and assigns.  Any

action or proceeding brought by either party against the other arising

out of or related to this agreement shall be brought only in a STATE

or FEDERAL COURT of competent jurisdiction located in King County,

Washington.