Science Experimenters Kit #1 - Second Edition	                        (Using Computers in the Classroom)

©1997 J P Fuller

Science Experimenters Kit#1 - Second Edition

Published 1997

First Published 1990

J. P. Fuller

48 Portmarnock Circle

MANDURAH

Western Australia  6210

ph (08) 9535 6472

http://www.southwest.com.au

email: jfuller@southwest.com.au

© 1997  J P Fuller

ISBN

0 646 16384 1

This material is provided as part of the Robotics Kit and is covered by the same site license conditions. Duplication is permitted for licensed sites.

Contents

Forward/Acknowledgments 								4

Introduction											5

Input 									 		8

Characteristics of the Games Port						10

Reading the Games port								11

ANDing										12

Software											18

Output											20

Voltage Input										21

Using Computers in the Science Room

Typical Circuits										25

Light											26

Sound											30

Temperature										31

Magnetism										32

Motion Experiments									34

Appendix A   Where do you obtain more information?				37

Appendix B    Construction hints							39

Appendix C    A few more ideas							41

Appendix D    The Next Step?   A Users Group?					42

Appendix E    Notes About Software Included With This Kit			43

Some Final Comments									47

Forward

The material contained in this booklet relates to using "IBM-compatible" computers equipped with a standard "Games Port". If your machine does not have a games port, one can be easily added for around twenty dollars. Some compatibles are supplied with a games port, but it is 'disabled' to avoid possible conflict with other cards you may subsequently add. When you order your system, make sure you specify that the games port is "enabled".

There are a number of manufacturers who offer plug-in 'control' boards for the 'compatibles', but the cost and complexity usually limit their usefulness in schools. The Games Port approach may not be as accurate as other techniques, but it is easy for students to understand and VERY cheap!

The approach outlined in this booklet will allow schools to introduce quite sophisticated "real-world" sensing for very little outlay (around $20 per machine).

It is my hope that the material presented will be sufficiently practical to be of direct use to teachers. Unless you are prompted to manufacture some of the circuits outlined and use them in your classes, my efforts will have been wasted.

Thanks and Acknowledgments:

The components and other supplies needed for this Project were made available through a grant from the Armadale Education District's 1990 "Minor Education Programme".  I thank the committee for its continuing support for this avenue of investigation.

Thanks again for the programming support of Mike Leishman from Balga SHS. Much of Mike's advice and a  number  of his Pascal 'tricks' are incorporated into this work.

Thanks for the continuing 'technical' support of Reg Ion, retired technician from Mandurah.

Thanks finally to the many Electronics Australia contributors who prompted the design of a number of the interfaces used in this work.

Introduction

Real World Interfacing of Computers:

General:

An 'interface' is a device which allows connection between the computer and the outside world. In most cases, the interface must modify the 'signal' from each to make it acceptable to the other.

You could, for example, use a computer to turn a light on and off directly, but the current drawn by even a small globe is likely to damage the power supply of the computer. In the case of lighting up a globe, the 'interface' would typically be a simple circuit used to 'drive' a relay, which in turn switches the larger current needed by the light globe.

Alternatively, you may want your computer to 'read' light intensity, or temperature changes. The 'interface' in this case would be the circuitry needed to 'block' any source of current entering the computer, while still allowing it to detect the desired changes.

In each case, the nature of the 'interface' is different, but the same principle of ensuring 'compatibility' applies.

This booklet details a number of 'interfaces' which may be constructed by (skilled) students and used with the 'compatibles' for 'real world' control and sensing. This work is most definitely NOT a book of theory. The aim is to bring together the information needed to construct and control such devices, without expecting deep technical knowledge on the part of the user. (Us!!)

Introduction

Design Pre-requisites:

There are a number of companies who specialise in 'control' boards for the 'IBM-compatibles'. Reference to most electronics and computing magazines will provide the names of these companies. If all you want is a single 'demonstration' unit, a commercial board may well be the best alternative.

My aim in this work is to provide simple, inexpensive designs which make it economical to consider large numbers of machines used in this way.

At the outset, it was decided that any circuits presented should be:

1.  cheap

2.  simple to construct using existing school facilities.

3.  constructed using readily available components.

4.  'external' to the machines, rather than 'plug-in' boards.

5.  simple enough in design for non-electronics people to understand (given a 				little help) - The aim being to de-mystify the techniques used.

6.  cheap.

As a 'rule-of-thumb', only those circuits which have actually been produced using existing facilities available in most secondary schools and purchased readily from local retailers are be included in this work. The software used was Turbo Pascal Version 5.0 . With the exception of some "Graphix Toolbox" procedures and one or two routines which have been "assembled" to provide acceptable speeds, all of the software presented will be 'text book' Pascal. (BASIC could be used in most cases, but at the expense of speed.)

I hope you will find the material presented to be easy to follow and of direct use in your classrooms.

Introduction

Interfacing Techniques:

There are a number of ways of 'reading' data directly from the outside world into a computer. In most cases the technique centres on a special-purpose board plugged into one of the machine's "expansion slots". This approach can not only be very expensive, but also results in a deepening of students' feelings of mystique surrounding computers in general. One of the aims when using computers in classes must surely be to enhance understanding, rather than further cloud the issue.

To help in the de-mystification process, any circuitry used should be 'exposed' for the user to see. Opaque 'project' boxes may make the finished product a little neater, but they hardly help the child (or adult!) to appreciate the simplicity of what they are doing. A clear 'perspex' box, or even no box at all would be preferable.

With simplicity in mind, the problem is to produce a suitable interface for science activities which is both inexpensive and accurate enough for reliable data acquisition.

As chance would have it, just such an interface already exists on the 'compatibles'.  It appears that the 'clone' designers followed Apple's lead in including a "Games Port" in their PC. The Games Port provides for the connection of two "joysticks", each with two switches and two potentiometers. This configuration is ideal for experimentation. The 'switched' lines may be used for simple on/off sensing and the 'resistive' lines allow Analogue to Digital conversion.

The port is robust and easily accessed via a 15-pin connector mounted at the rear of the machine.   -   See Fig 1 for details on the Games Port.

(You may have problems actually getting the plug into the socket on some 'compatibles' - the designers were a bit out in their tolerances!)

NOTE: If your present (or future) machines don't have Games cards, you should consider investing in them. This approach to 'interfacing' is one of the cheapest! (A games card will cost you only around $40 per machine.)

INPUT

INPUT:

(The information below is reproduced from the previous booklet called "Robotics#2 "- See Appendix A for information on ordering this booklet.)

The Games Port is the most convenient means of reading data into the 'compatibles'. Four 'switched' lines and four 'Analogue to Digital' (A to D) lines are available via a 15-pin socket at the rear of the machine.

THE GAMES PORT

Games Port Schematic					Games Port Pin/Socket 											numbers Viewed from 											solder side of Plug.)

INPUT

GAMES  PORT  PIN  SPECIFICATIONS

PIN NUMBER                       		   FUNCTION

1                                		+5 Volts

2           	D4                 	Button 4  (Switched)

3           	D0                 	Position 0  (A to D)

4                                		GROUND

5                                		GROUND

6           	D1                 	Position 1  (A to D)

7           	D5                 	Button 5  (Switched)

8                                		+5 Volts

9                                		+5 Volts

10          	D6                 	Button 6  (Switched)

11          	D2                 	Position 2  (A to D)

12                               		GROUND

13          	D3                 	Position 3  (A to D)

14          	D7                 	Button 7  (Switched)

15                               		+5 Volts

The "ADDRESS" of the Games port is 512 decimal. ( 200 HEX )

PCB Pattern			          Data line Connections

INPUT

Characteristics of the Games Port:

This work attempts to keep things as simple as possible and demands only minimal electronics knowledge on the part of the reader, but there are occasions when a little bit of 'technical' information can be helpful. If you find this section beyond you, don't worry, it is not essential that you understand it all.

There are two integrated circuits ("ICs") at the Games Port which carry out all of the interfacing we make use of.

Switched Lines -

The four 'switched' lines feed into half of a 74LS244 IC which acts as a "buffer", protecting the internal circuitry of the computer and 'squaring-up' the input signal to ensure reliable detection by the CPU. Each of the switched lines is held 'high' by an internal resistor to prevent false triggering. The 'sensor' you use must therefore switch the line to 'ground' to produce a change in the state of the line.

Analogue to Digital Conversion -

The 'analogue' lines feed into a NE558 "Quad Timer". Many of you will be familiar with its smaller 'cousin'; the NE555. The NE558 contains the equivalent of four  NE555  'modules'.

Each '555' module is used in "monostable" mode - ie when a pulse is detected at the 'trigger' pin, its output goes high for a duration determined by a capacitor/resistor network. In the case of the Games Port, the "capacitor" is internal and the "resistor" is what we attach to the port.

The triggering "pulse" is sent to the port by the CPU when it issues a 'write' command to the address of the port  -  ie Port[512] := <some value>

Each output line of the NE558 feeds into the second half of the 74LS244 referred to above, for 'buffering' purposes.

Software then sets up a 'timing' routine to determine the duration of the 'high' state of each line of the NE558 and relate this to the value of the "resistors" connected to the port.

INPUT

Reading the Games Port:

The Games port may be 'read' using BASIC commands such as:

100  V = INP(512)          OR:   V = INP($200)  (HEX)

200  PRINT V

In Pascal, the syntax is :

V := PORT[512];              OR:   V := PORT[$200]; (HEX)

WRITELN(V);

In both cases above, the value returned for "V" reflects the HIGH, or LOW state of each pin at the Games port.

eg 	If V = 128, Data line D7 is HIGH (ie, the switch is open) and

all other lines are LOW.

If V =  45, Data lines D5, D3, D2 and D0 are HIGH.

( 45 = 32 + 8 + 4 + 1 )

Software must then interpret the value returned as "V" to give the actual HIGH/LOW  state of each line.

NOTE:  Nomenclature Convention - In the following pages the

term  "High  State  Value"  is used to refer to the decimal

equivalent of each 'place' in an 8-bit byte.

eg   Place8    Place7    Place6  Place5   Place4    Place3   Place2   Place1

128       64             32         16          8            4           2           1

"High State Values"

INPUT

Decoding Individual Pins:

The state of the 'switched' data lines appears on the four 'higher' data bits of the byte returned by "V:=Port[512]" ie the High State Values: 128, 64, 32 and 16) The four 'lower' bits return the state of the 'A to D' lines.

In order to decode the state of a specific line the technique of "ANDing" is used.

ANDing:

Forgive me if you are familiar with this technique, I found it difficult to grasp, so hopefully someone else will appreciate the refresher. -

The operator; "AND" is available in both BASIC and Pascal. Each usage is slightly different, so you can choose the  approach which best suits you.

In its simplest use, the result of ANDing two binary numbers is indicated below:

11001100 AND 00100100        is:       00000100

(ie: if both corresponding 'bits' are 1, then that bit in the result, is also 1.)

ie.   The "AND" operator returns a TRUE if both bits are TRUE (ie: "1").

SOME MORE:

11000100				11101111

AND  01011011          		AND  11110001

is:    	01000000               	is:    	11100001

REMEMBER:         A "1" means 'HIGH', or +5 volts DC.

INPUT

When decimal (or HEX) values are used, the results are not quite as obvious, but if you perform the binary conversion, it really is as simple as indicated above.

In  BASIC,  the AND  operator returns  a logical TRUE, or FALSE,  depending  upon whether the result of  ANDing is 1, or 0. (respectively)

For example:

"134 AND 4"    ......is:    TRUE

(134 = 128 + 4 + 2)

"134 AND 8"    ......is:     FALSE

Hopefully, you can begin to see the technique used to interpret the input obtained from a command such as "V := Port[512]" into the HIGH/LOW state of each data line.

In essence, the procedure involves:

- read the input value.

- AND this value with the High State Value of the line

in question.

- if the ANDing is TRUE then that line is HIGH, or ON.

eg. If we want to know if D6 is HIGH:

If (input AND 64) is TRUE then

D6 is HIGH

else

D6 is LOW

INPUT

Some More Examples of ANDing:

(Data Lines)

Operation	            	D5     D4     D3     D2    D1    D0

3 AND 2 = TRUE          	(3 =                              		2   + 	1)

4 AND 2 = FALSE         	(4 =                        	4   + 	0   + 	0)

24 AND 8 = TRUE	(24 =            16  + 	8  + 	0   + 	0   + 	0)

33 AND 1 = TRUE         (33 =  32  +  0   +   0  + 	0   + 	0   + 	1)

Since the 'switched' lines at the Games Port are on the four 'higher' bits, you need to AND the value read by "InValue:= Port[512]" with 128, 64, 32 and 16.

Inverting:

There is one small complication, in that the 'switched' data lines are held HIGH by internal resistors (see Fig.1). When a switch connected between one of the lines and 'ground' is closed, that data line goes LOW. The effect of this situation is that input values read from the Games Port are opposite what would be expected. If, for example, the switch on line D6 is closed, the value read would be (say) 191 rather than 64! This means that before ANDing the input value, it must be subtracted from 255 to invert the value and thereby simplify the decoding procedure.

(You could perform ANDing with the 'un-inverted' values, but you would need to think in the negative and deal with FALSEs, rather than TRUEs.

I prefer the former, so that is the approach I will adopt.)

cont over..........

INPUT

The necessary code therefore becomes:

InValue := 255 - Port[512];

if (InValue AND 16) = 16 then

WRITELN('The switch on data line "D4" is CLOSED.')

else

WRITELN('The switch on data line "D4" is OPEN.');

In BASIC, the TRUE/FALSE condition is used, along the lines of:

140  D4$ = OPEN

150  V = 255 - INP(512)

200  B = (V AND 16): IF B THEN D4$ = CLOSED

300  PRINT "The switch on data line D4 is ";D4$

In each of the above examples, the 'AND' technique is used to decode the state of each data line by comparing the value returned from the Games port (Address: 512) with the "High State Value" of the line under examination. In this case, data line four (D4) is tested.

If the switch connected to D4 is closed, "V" will 'include' a value of 16, indicating that the D4 bit is "1" (after the inversion takes place!). (The actual value of "V" can be anything up to 255, as long as it 'includes' 16).

The other 'switched' lines are decoded by 'ANDing' "V" with 32, 64 and 128.

NOTE the different way BASIC and Pascal refer to the outcome of ANDing.

With Pascal, the result of the AND operation is tested against the expected integer value using "=". (NOT the assignment operator  ":=")

The BASIC syntax    ".... :IF B then D4........"   in line 200 means: if B is TRUE then ...........<action>..........  In other words: if the comparison of the inverted input value AND binary 00010000 is 00010000 then  ....<action>.....)

INPUT

INPUT:  (Using the Games Port)

The 'switched' data lines are relatively easy to decode. The previous section provides information about the basic techniques involved.

The Analogue to Digital ("A to D") lines are, however, a little more complex.

The Games port has a built-in 'A to D' converter which 'tests' resistors connected across each of the four 'lower' data lines and returns an integer proportional to the resistance. The commands are:

Pascal -	InValue := Port[512];

BASIC -      InValue = INP(512)

You may, for example, return a value of 10100110 (Decimal:166) for "InValue". This tells us the following:

1	0	1	0		0	1	1	0

0N    OFF  	ON  	OFF   	OFF    ON   	ON  	OFF

Switched Lines               	    Analogue Lines

Before 'reading' the AtoD lines at the Games Port, you must send a control signal to the 'A to D' converter (see Fig 1.) to initiate the decoding procedure. (Any value may be written to initiate this process. It is the 'WRITE' signal itself which sets up the Port, rather than the actual value written.)

Once this signal is received, each of the four lower bits goes HIGH and remains HIGH for a duration determined by the value of the attached resistor. (Between 0 ohm and 100kohm.)

continued over …/

INPUT

A to D conversion      ....cont

The 'control signal' referred to above is:

Pascal - 	Port[512] := <any value>

BASIC -      OUT 512,<any value>

The sequence for reading the 'A to D' lines therefore becomes:

- Send the control signal.

- Read the port.

Set up a counting LOOP until each line goes LOW.

- Interpret the 'count' as a resistance value.

A typical Pascal routine would follow  along the lines of:

Count := 0;   		{ Initialises the loop counter.}                                                                Port[512] := 1;          {  Starts the 'A to D' process. Any value would do.}

repeat

PortByte := Port[512];     { Reads the Games Port}

count := count + 1;

until (PortByte AND 4 = 0) OR (count > 10000);

{ Tests to see if D4 is still HIGH and stops }

{  the loop before MaxInt is reached.        }

WRITELN(count);

Having obtained a 'count' for a data line, the software must then relate this 'raw' value to resistance.

NOTE: The example given above examines only ONE of  the four data lines. If  you want  simultaneous conversion  of all lines, you would  need to 'AND' "PortByte" with 1, 2, 4 and 8 while 'counting' during the HIGH state of each!

SOFTWARE

Sample Software:

Included in this Kit are a number of Pascal programs which demonstrate various decoding techniques. (See page 25 for a listing of programs provided with this 'Kit'.)

If you tried the 'A to D Conversion' outlined above, you were probably disappointed with the resolution available due to the relative slowness of the counting loop. Even Pascal can be laboriously slow! BASIC is completely out of the question for any serious sensing.

Mike Leishman from Balga SHS has written an Assembly routine to solve this problem for Pascal users. To use this procedure, the file "ADA.OBJ" must be on driveA: .

He has written this routine as a "Function" which may be called at any time during your own program.  Mike's function is defined as "ADADCODE" and is declared in the form given below:

FUNCTION ADADCODE(MASK:INTEGER) :INTEGER;EXTERNAL;

{Written by Mike Leishman}

{ This function is 'external' to this program and may be called

in the  form: VariableName := ADADCODE(Num)

(Where Num =  1,2,4,or 8) }

A typical usage would be:

Line1Value := ADADCODE(1);

The value returned for "Line1Value" will be an integer from 0 to just under 5,000, as determined by how far the function 'counts' during the HIGH state of Data Line 'D-Zero'. This time interval being proportional to the resistance connected between pin 3 and GROUND.

(It would be appropriate to acknowledge Mike's contribution, in programs  making use of this function.)

LATE UPDATE: Pascal 5.5 doesn't like the form of the OBJ file. Compile your finished program in version 5.0

SOFTWARE

Trying Out Your Software:

Once you have constructed your Games Port interface (from page 9) it is time to put it to the test.

The simplest application of the 'A to D' feature of the Port is to connect a potentiometer (variable resistor) across one of the lines as shown:

The resistance between pin 13 and ground can be 'read' using Mike Leishman's "ADADCODE" function, or a Pascal routine like the one on page 17. If it is important to obtain exact resistances, rather than relative values, you will need to calibrate the readings obtained by comparing values read by the computer with measurements made using an accurate meter.

WARNING....  Disconnect the computer before using the meter -	most  have  an  internal  battery,  which  could cause problems!!

If you want to experiment with 'ready-made' demonstration software, copies can be obtained as part of the 'site licence' for the "Robotics" manuals, or you could use the software supplied (to 'registered' sites) with this booklet. - See Appendix A for details.

Output:

The principles used in controlling devices connected to a 'centronics' printer port were outlined in the booklet; "ROBOTICS". If you have not yet read and digested the relevant sections in that work, you would be advised to do so before continuing. (See Appendix A for how to obtain a copy.)

OUTPUT:  (Using the Printer Port Interface )

Output control is achieved by sending the appropriate 'code' to the printer port. If, for example, you wished to 'turn on' lines D0, D1, D2 and D3, the command would be:

Pascal -		Port[888] := 15;

BASIC -      	OUT 888, 15

(15 = 1 + 2 + 4 + 8)

The 'interface' used to drive devices such as motors and globes, is based on the circuit below:

(See the "Robotics" booklet for details.)

Up to eight such 'drivers' may be connected to the printer port.

NOTE: You must NOT use the computer's internal power supply to drive these devices. An external supply must be provided!

VOLTAGE

Voltage Input:

As a general rule, you should never input any current into a computer.

While a 'short circuit' in your sensing equipment might cause a few problems, if you were to actually apply voltage to one of the computer's ports, you could cause massive internal damage!!

A number of different techniques are used to enable a computer to detect changing voltage. Most of them involve some form of "Voltage Controlled Oscillator" (VCO) and software to interpret the change in pulse rate (frequency) as the applied voltage changes.

Following the themes of SIMPLICITY and LOW COST, it was decided not to use the 'VCO' approach in early experimentation. Both the hardware and software used with this type of A to D conversion would tend to make the process too complex for our purposes.  (Kit#2 will look at VCO's.)

Using an 'Opto-coupler' -

By now, you should be familiar with the 'resistance-measuring' option at the Games Port and hopefully, you have had a look at some of the software used for decoding. Since the 'opto-transistor' worked so well for reading light levels, it seemed reasonable to use some sort of opto-coupler for voltage/resistance conversion.

VOLTAGE

Voltage Input cont....

The system above would probably work reasonably well, but there would be problems with ambient light and critical factors such as the distance between the globe and detector.

All these shortcomings are overcome by the use of an "Opto-coupler" integrated circuit.  This unit comes as a small device with the light source and detector built in. A typical example is the six-legged 'dil' package "4N28" IC, which costs around $2.00. The breakdown voltage between the source and detector is in the region of thousands of volts, so you won't need to worry about voltages 'getting into' your computer.

VOLTAGE

Voltage Input cont....

Using the 4N28:

If too much current flows through the LED inside the 4N28, it will be destroyed. As a general rule of thumb, you may consider it to be a 1.5volt globe. We all know what happens if you connect a higher voltage to a torch globe, well the LED will be similarly effected.

If you intend using voltages higher than 1.2volts, you must include a 'dropping resistor' in series with the LED to limit current -

Using LEDs with voltages > 1.2volts

If you are unsure as to whether your current source is suitable for connection to the 4N28, try it first with an ordinary LED (cost approx 20cents). If the cheap LED survives then your opto-coupler IC should also survive. (We hope!)

As a guide:

6v use  220 ohm

Values for		9v use  390 ohm

"R":  	       	12v use  560 ohm

24v use 1.2k ohm

(From the Dick Smith Catalogue.)

VOLTAGE

Voltage Measurement with the 4N28:

If you've tried an MEL 12, or FPT 100 photo-transistor connected to one of the 'A to D' lines you will have already seen the dramatic swing in resistance as light intensity changes. This principle is applied to the 4N28 by varying the current to the internal LED, which in turn determines the amount of light emitted and therefore; the resistance of the output transistor. The computer then reads the changing resistance of the internal transistor and relates it to the original current driving the internal LED.

To use this approach, you must first determine the maximum voltage which will be applied to the 4N28 and select the value for a 'dropping' resistor to limit current through the LED (see page 23) . The example given on page 22 uses a 10k½ resistor and has been found suitable for input voltages up to about 10 volts. Trial and error will enable you to find values suited to your own particular application. (Just make sure that your dropping resistor does not fall below the value calculated as per page 23!.)

The circuit below can be used to test and calibrate readings obtained using the 4N28. Start off with  a few volts and gradually increase the potential applied across pins 1 and 2.

Make sure that the 'dropping' resistor is large enough to prevent damage to the internal LED of the 4N28!!

Circuits

Using Computers in the Science Room:

Now  that  you have constructed and tested a number of  interfaces for  the  'compatibles'  it  is  time to look  at  ways  to  use  these techniques for data acquisition and control.

Software:

A  number of sample programs are provided with this  'Kit'.  Feel free  to modify them for your own requirements.  The simplest  of these  detects  the  on/off state of one of the  'switched'  data lines at the Games Port.  Decoding the 'resistive' lines requires much  faster  routines such as Mike Leishman's  "ADADCODE"  which uses an 'assembled' function called by the main program.  Mike is quite happy to have us use his work in this way,  but it would be appropriate to acknowledge his efforts when you do.

Once you have grasped the techniques of reading data at the Games Port  the  next step is to store and present the information  for use in class situations. Sample programs are provided on the disk accompanying  this  booklet  which  demonstrate  techniques   for storing  data in 'array' structures,  displaying data directly to screen in graphical form and how to obtain 'hard-copy' print outs.

Programs supplied as part of this 'Kit' are:

KITMENU1.EXE - Displays the menu used to select from ...........

EXPKIT_7.EXE -   Displays, stores and prints data read from the Port. (NOTE: This 	program makes use of "Hercules" drivers and routines the Turbo Pascal Graphix 	Toolbox.)

GAMEPLS3.EXE  - Detects and records 'pulses' read at the Port.

INGAMES2.EXE -   Displays the state of the Games Port lines.

READ_ANA.EXE -  Reads and displays the state of the 'analogue' lines.

SWITCHED.EXE -   Reads and displays the state of the 'switched' lines.

TIMER2.EXE -        A simple 'stop-watch' program.

(See Appendix E for more information about this software.)

Circuits

Typical Data-Acquisition Systems:

Light Intensity -

Connect  an  FPT100 Opto-Transistor between Ground and one of  the 'analogue'  lines at the Games Port and use ADADCODE to read  the change in resistance as light intensity varies.

You  will  need to calibrate the system using a commercial  light meter if you need absolute readings,  but for most  applications, relative intensities will suffice.

Opto-Transistors  have  three 'legs',  but only two are used  for most  applications;  the  emitter and  collector.  (The  base  is sometimes  used  to  de-sensitise  the circuit  by  reducing  its 'gain'.)

Make sure that you connect the FPT100 as shown, or you will produce strange readings from ADADCODE.

NOTE: The ORP 12 Light Dependant Resistor("LDR") can be used  instead  of  an  opto-transistor.  ( An LDR has  a much slower response  time and may  not  be suitable where fast switching is required.)

Circuits

Spectral Analysis? -

The  imaginative  use  of  filters  should  provide   interesting discussion  about spectral analysis and automated systems such as used in chemistry and astronomy.

As  a  typical example,  set up a suitable source of  ultra-violet light and two opto-transistors, one covered with and ultra-violet filter  ( such  as Kodak's KD16-150, available at  around  $15 per 25 square cm from Perth Professional Sales and Services. )  You  may then use the  differential  between  the 'base' readings of each detector to determine the  "U/V-blocking" power of material held over the 'exposed' detector.  You can  now use  this system to,  for example,  measure the effectiveness of various  'sun  screens'  smeared onto glass  and  held  over  the sensor.

Once again,  this will only provide relative readings,  but these will probably have just as much meaning to students as technical, absolute units.

LATE NOTE:   I have had difficulty sensing U/V light. Use the same technique, but change the source/detectors to Infra-red!

Circuits

Spectral Analysis? cont....

Coloured filters may be used in the same way to give the computer the  capability  of differentiating between colours in  a  'mock' spectral analyser, where the resistance of the opto-transistor is determined  by the type of filter and the colour of the  incident light.

(You  will  need to use slightly different logic in the  case  of colours,  because  coloured  'filters' are designed to  let  that specific  colour through,  rather than block it out,  as with the U/V  filter  discussed above.  - In this case we can  use  all  four 'analogue'  lines  and  four different filters  to  identify  the colour by elimination.)

It may be an interesting exercise to use this technique with 'sun-glasses' worn by students. Both U/V protection and 'tinting' can be compared.

With a little creative design, it should be possible to modify the procedure to enable reflectivity to be examined as well as transmission characteristics.

Circuits

Measuring Turbidity -

This  is a relatively simple application of the  opto-transistor, but how about applying the technique to chemistry experiments?

You may , for example, like to try this technique for automated timing in the 'clock' reaction using sodium thiosulfate and hydrochloric acid.

If  you've ever looked inside one of those very expensive  'Water Test' kits you will have discovered that they consist simply of a globe  and opto-sensor,  much like our own  arrangement.

With  a little  encouragement,  I'm sure that students could come up with ideas for an automated test kit!  With the exception of pH, all of the water test kit's functions can be easily duplicated by the computer.

The use of computers for this type of automated environmental 'logging' of data should open up wide possibilities in the minds of students. Such data can be collected 24 hours a day without the need of direct supervision. Once collected, data can be exchanged between groups in other schools, states and even internationally!

Circuits

Detecting Sound:

In its present form, this circuit has limited use in the science room, but it may trigger some ideas in your own mind (that I can't think of!). In his work relating to interfacing the 'Apple' Games Port, Richard Walding suggests using this type of circuit for detecting 'nodes' in sound experiments using air columns. (See Appendix A for more information on Richard's work.)

The circuit presented produces a voltage which varies in relation to detected sound. It is not very sensitive, but it does represent another tool in the armoury.

The "Electret" microphones vary in price from 50 cents to several dollars, depending on where you get them. There also seem to be a few versions, so a little experimentation may be needed.

The information below should get you started...

NOTE:   You may use the Games Port power supply to drive  this board directly.  Connect the "Ground" from this board to the "Ground" of both the computer AND the '4N28' board.

Circuits

Temperature Changes:

The simplest method of sensing temperature change is by using a "Thermistor" (sometimes spelled: thermister?). The most common varieties are the "1k ohm and "10K ohm types costing around $1.50 each.

These devices work by changing resistance as the temperature changes. Usually, the  change in resistance is too small to be used with one of the AtoD lines at the Games port, so the technique of 'voltage' detection through the 4N28 is employed (as with 'sound', on the previous page.)

A 'Darlington pair' arrangement is used to greatly 'magnify' changes in the Thermistor and produce a relatively larger voltage swing. The voltage is then 'read' via the 4N28 interface.

The basic circuit needed is:

Measuring Temperature

NOTE: You may need to experiment with the 100 ohm resistor. The actual value needed depends upon the size of the resistor used with the 4N28.

Circuits

Detecting Magnetic Fields:

Using the UGN 3020 Hall Effect IC -

This IC responds to magnetic fields by switching its output from High to Low (and vice versa). The diagrams below should provide sufficient information for you to make use of this device:

The actual values for the resistor and capacitor are not critical. The manufacturer recommends 820 ohm and 20pF!

Since the IC has a built-in regulator, there is no need to worry about providing any external voltage regulation. Power may be drawn directly from the Games Port, or via an external supply.

cont over.......

Using the UGN 3020 Hall Effect IC   cont....

For my experimenting, I used an "Alnico" bar magnet of the type usually supplied to school Science Departments. When the south pole of the magnet was brought near the 'branded' face of the IC its output switched rapidly from High to Low. - According to data sheets, a switching frequency of up to 100kHz could be expected from this device.

Unfortunately, this is not the most sensitive of devices. The magnet needs to be within a few mm (Yes..millimeters!!) to have any effect. It is however, wonderfully simple to use and very economical at around $1.50 each.

Commercially, the 3020 would typically be used to find the RPM of spinning shafts. Small magnets would be fixed to the shaft and the IC mounted a small distance away. As the shaft rotates, pulses are generated. (In such an application the small gap required between the magnet and the device is no problem - the designer could be sure that the shaft would not 'wobble', at least under normal operating conditions!)

Outlined below are a number of experiments you may like to try:

Circuits

Motion Experiments -

The 'compatibles' can be used for all the 'event' experiments carried out presently using the air track in conjunction with electronic 'timers'. The advantage of using the computer, rather than a 'timer', relates more to motivation and expanding the student's experiences than to any perceived increase in precision.

The computer can, of course, be programmed to perform all the necessary calculations and simply present students with final results of times, changes in momentum, etc, but it is debatable whether this leads to increased, or decreased, understanding of the physics involved!

Using computers in physics experiments can certainly enhance learning and stimulate the student's imagination, thereby  providing greater awareness of the 'real world' of data-gathering and, perhaps, motivating them to find out more about how 'real' scientists perform their experiments.

As a general rule:    If the computer enhances learning, it should be used.

If it is just a gimmick, it should not be used!

A Typical Air-track Setup -

cont over..........

Circuits

Motion Experiments    ........cont

Event Timing:

Using a light source and an MEL 12 opto-transistor connected to one of the 'switched' data lines, the computer can be used to time 'intervals' such as a falling object in 'gravity' experiments where the object passes between two sensors; one to start the timer and one to stop it.

Event Timing

If you would like to experiment with this setup, you can use the program called "GAMEPLS3.EXE", with both opto-transistors connected to the same line. (The program is set up to read D4 at the moment.) This program reads and records the ON/OFF durations of line D4.

In this case, it will provide you with information about how long each opto-transistor is 'activated' and the time between events. You will need to do some calibration and manipulation of the data, but at least it will get you started.

cont over............

Circuits

Motion Experiments    ........cont

Duration Timing:

It is a simple task to set up a counting loop to determine how long a switch is closed. This technique can be used with, for example, the air-track to determine velocities using a 'card' of known length.

Duration Timing

The 'stop-watch' routine called "TIMER2.EXE" may be used to record the time for which the data line is 'low'. This program will give readings to the nearest 1/100th of a second, so you should be able to produce some quite respectable results! (TIMER2.EXE is  set up for Data line D4, ie pin 2.)

Once you have a figure for the time D4 is Low, it is a simple task to calculate the velocity of the 'slider'.

With a little imagination, it should be possible to use "GAMEPLS3.EXE" with two sets of opto-switches to gather data on before-and-after momentum changes in 'collision' experiments using the air-track.

APPENDIX A

Where do you obtain more information?...

This work is the third in a series dealing with general interfacing of computers in W.A. schools. The first two booklets; "Robotics" and "Robotics2" were completed during 1989/90. Those booklets dealt with interfacing techniques for the 'compatibles' and Microbees.

Sufficient information is provided to allow schools to manufacture their own simple interfaces for use as a foundation of any 'Robotics' studies.

The Robotics "set" also includes demonstration software as both 'executable' files and Turbo Pascal V5.0 source code.

Schools purchasing a 'site licence' for these booklets are permitted to duplicate the work freely, for use at the registered site.

All the text material is provided 'on-disk'  in Microsoft Word,  for schools to print out using their own facilities.

To obtain 'site rights' to Robotics, send an official school order form to:

Jim Fuller

Computing Department

Mandurah Senior High School

Gibla Street

MANDURAH   W.A.   6210

phone: (08) 9535 3800

FAX: (08) 9535 2966

email: jfuller@southwest.com.au

NOTE: This is a fund-raising project for the School,  ALL  money raised by the sale of site licences is paid into school funds. The author receives  no remuneration either directly, or indirectly.

APPENDIX A

Where do you obtain more information?  cont...

Richard Walding, Senior Physics and Chemistry Master at Moreton Bay College in Queensland, has put together a very useful package on interfacing the Apple (not Macintosh!) Games Port. The characteristics of this port are very similar to the 'IBM' counterpart (with the exception that Apple's can output as well as input).

He details the specific characteristics of the components used and outlines a series of activities aimed at upper school Physics classes. For those of you with Apples, he also provides, on disk, all relevant text files and the routines needed to run his suggested circuits.

Even if you don't have access to an Apple, this material will prove very useful. It is not easy to find material about interfacing which is both cheap and clearly explained. Richard's work is both!

Copies of his work (called: "ELAINE") are available on a site-licence basis for $69.

To obtain a copy of ELAINE, send an official school order to:

Richard Walding

Moreton Bay College

450 Wondall Road

Wynnum, Queensland  4178

pH (07) 390 8555

APPENDIX B

Construction Hints:

Making Printed Circuit Boards -

The artwork for all the interfaces in this booklet are drawn to scale, twice actual size. To produce actual-size artwork, use the "50% Reduction" option from the "Page Setup..." menu.

You may then  either draw the circuit by hand, or take the artwork to a company such as Jemal Products in Welshpool to have the boards made professionally. (They will require a high-quality original of the artwork - usually produced on a laser printer!)  The cost of having boards  manufactured depends upon size and quantity. As a general guide, a board of about 10cm by 10cm will cost you around $5.00.

Etching Your Own PCBs -

The fastest way to produce 'proto-type' boards is by etching with a mix of Hydrogen Peroxide and 6M HCl. Make up just enough etchant for your immediate needs, because it tends to go off rather quickly.

WARNING - This etchant should not be used by students!!

Use standard Printed Circuit Board ("PCB") and draw your lines with an ordinary 'texta' (the water proof variety!).

You can purchase 'off-cuts' of PCB from Jemal for around $25.00 per Kg.

For prototyping small circuits, you may find Veroboard a better alternative. (Veroboard has strips of pre-drilled copper bonded onto a substrate.) Be warned however, that it is a nightmare to find faults in large circuits produced in this way!

APPENDIX B

Connecting to the Games Port -

I have found that 'Terminal Blocks' are ideal for the temporary connection of devices such as switches and light sensors. A small screwdriver is used to fix each wire into either half of the block. The component can be attached to one side of the block and wires coming from the 'interface board' (as per page 9) are then simply screwed into the other side of the block.

A 'ten-way' terminal block costs around $2.00.

You can use the whole strip with a number of devices connected at the same time, or cut it into smaller blocks for connecting individual components.

Using a 'Terminal Block' to Mount Components:

Mounting Individual Components

APPENDIX C

A Few More Ideas....

Falling Bodies:						Oscillations:

Expansion/Contraction

Velocity:

APPENDIX D

What Now?........

In carrying out research for this work, I made contact with a number of people who are similarly interested in computer interfacing and using computers in science classes. As outlined in the previous pages, there are a few commercial groups offering interface 'kits' and quite sophisticated software at reasonable prices. (Some are most definitely un-reasonable!)

I do, however, see value in "re-inventing the wheel". There is nothing quite as enlightening as actually finding solutions to problems. The insight gained by building simple interfaces and writing software to make use of your own creation is hard to compete with. Both teachers and students will find the experience well worth while!

I have some ideas in mind about what to attempt next, but I would welcome contact with others sharing an interest in this area. (As, I imagine, many of you would also!)

One possibility is to establish a "Users Group" of teachers for the exchange of information.

If you have any suggestions, or ideas for future developments, I can be contacted during the day at Mandurah Senior, or after hours on (08) 9535 6471 (MANDURAH).

Please Note:

The intention is to establish a mechanism for the 'free' exchange  of  information.  I  still  believe  strongly  in the ethic of helping each  other and working together.

Any charges involved with the distribution of this type of information would be limited to school fund-raising. No individual would derive personal gain from this venture.

APPENDIX E

Notes on the Software Included with this Kit:

The enclosed 5 1/4" DOS disk (supplied to 'registered' sites) contains the Pascal source code and 'executable' files needed to begin experimentation.

The files on the disk should include:

KITMENU1.EXE 			SCI_DATA.TXT

EXPKIT_7.EXE 			SCI_DAT2.TXT

GAMEPLS3.EXE              	ADA.OBJ  (Mike Leishman's)

INGAMES2.EXE

READ_ANA.EXE             (Plus various 'housekeeping' files

SWITCHED.EXE               used by the Graphics Toolbox.)

TIMER2.EXE

KITMENU1 - This 'master' program allows the user to select between the different data-acquisition programs on the disk. It is set up to provide seven choices, but only six are used. At the moment, entering '7' will produce a BEEP (write#7). If you want to add another program, change the 'case: 7' line on page 3 and add an 'exec' line to call your new program.

Make sure that you use the same format as used for the other six 'exec' calls. (In particular, make sure that the last characters in the 'exec' brackets are two single quotes, NOT a double quote!)

EXP_KIT7 - This program is the cornerstone of the software provided with this Kit. It is certainly not perfect, but it should demonstrate the techniques used to read data, display the information, draw graphs and store data to disk.

In essence, what the program does is:

- Asks the user for the time over which data is to be recorded.

- Accepts labels for the graph

- Reads one of the 'AtoD' lines (D3) using Mike Leishman's ADADCODE.

- Displays the data to screen (in 'graphics' mode) as a bar graph.

- Allows the graph to be printed.

cont over.....

Notes on the Software Included with this Kit:

EXPKIT_7	cont.....

- Displays the first 14 readings (in 'Hercules' mode).

- Allows the user to save ALL readings to the disk file called "SCI_DAT2.TXT. The data 	is stored in a format compatible with most Databases.

- Allows printing of ALL readings.

Notes on Using EXPKIT_7 -

The program expects to find a file called SCI_DAT2.TXT on driveA: .

If no file exists, the program will 'crash'.

Each time data is saved to disk, it OVERWRITES previous readings. If you wish to save the data from earlier experiments, the simplest approach is to copy the contents of the file to one of a different name.

eg: (From the DOS prompt)  A:>COPY A:SCI_DAT2.TXT A:SCI_DAT3.TXT

This will duplicate the file containing the 'old' data for later examination.

Printing:

You don't have to print out your results, but if you select the 'print' option, a printer MUST be connected and turned ON! If it is not, the program will 'crash'. (This isn't as catastrophic as it may seem, as long as you saved your data to disk before making this mistake.)

Using Turbo Pascal's "Graphix Toolbox" :

EXPKIT_7 makes use of the 'Toolbox' procedure called "Hard Copy" to obtain a print out of the graph. This routine is the graphics equivalent of "PrintScreen" using DOS. This appears to be the only way of producing a hard copy of a 'graphics' screen. (If you know of others, please let me know.)

Even Borland admits that the Graphix Toolbox is not "100% compatible" with the usual 'graphics' mode in Turbo Pascal. It is essential that any of your own "uses" declarations follow the 'order' used in this example.

APPENDIX E

Notes on the Software Included with this Kit:

TIMER2 -  This program is relatively simple. The user is asked to press a switch connected to D4. The program 'reads' the system clock at switch-closed and then again when the switch is released. The elapsed time is converted to hundredths of a second and displayed to screen.

The 'GetTime' procedure should prove to be very useful in your own experimentation and software development.

READ_ANA - This program again makes use of Mike's function; ADADCODE. It reports the 'values' of the resistances connected to each of the AtoD lines. If you want to obtain quantitative results you will need to 'calibrate' the readings using resistors of known values.

( The 1%- tolerance, 'blue' resistors, should do for most applications.)

Even if you only want to 'read' one of the lines, it is advisable to connect resistors to each of the lines, or the program 'loops' will slow things down too much. (A value of about 1k½ on each 'spare' line should do.) You will also need to do this for "INGAMES2".

SWITCHED - This program displays the ON/OFF state of each of the 'digital' lines at the Games port. It is set up to 'loop' repeatedly until a key is pressed. The looping results in considerable screen 'flicker'. If you want to eliminate this flicker, remove the 'curly' brackets around the 'DELAY' line on page 2 of the program. This will clean up the screen, but at the cost of sensitivity.

GAMEPLS3 - This is the 'standard' Pascal version of what I hope will become an 'assembled' routine to determine frequency changes at the Games Port. In its present form it is a bit too slow for this purpose, but it should prove useful where you want to detect a series of pulses and record data about the length of each 'pulse'.

Earlier in the booklet it has been suggested that this program can be used in 'momentum' experiments, where a number of events must be timed. This program eliminates the need for duplicate 'lines' using a number of sensors. Multiple readings can be made using just one data line!

cont over...........

APPENDIX E

Notes on the Software Included with this Kit: 	cont ....

GAMEPLS3

This program expects to find a file on driveA: called "SCI_DATA.TXT". If this file is not found, the program will 'crash'. (The comments above about EXPKIT_7, relating to 'copying' files for backup purposes, also apply here.)

The program only displays the first 14 readings to screen, but it continues to record data (up to 100 records) after the screen display is 'full'. You are then given the opportunity to print out the complete set of data and save it to disk (see above). As with EXPKIT_7, if you instruct the program to print, without a printer connected and 'on-line', the program will 'crash'!

INGAMES2 - This program was developed as part of "Robotics#2". It is included here for those of you who do not have copies of the Robotics 'set'. INGAMES2 provides on-screen information about all lines at the Games Port. It is very useful when 'prototyping' and testing your interfaces.

Make sure that you have at least 1k½ resistors connected across each of the AtoD lines, or the program will be slowed down to unacceptable levels as it 'counts' up to the maximum set by ADADCODE.

This program has a built in 'DELAY' to reduce screen flicker. If you want to speed things up a bit, remove the "DELAY(200);" line from page 5 of the program.

Please Note: I don't claim to be an expert. I can only  hope  that this material will be of some in your early  experimentation.  If you find any error come up with anything of value,  please let me know, that I can include the information on future disks.

Some Final Comments:

If you are using this material in your school, it must be 'registered'.

I have no objections to individual teachers obtaining copies of this work for perusal, but its use is limited to those schools who have paid for 'site rights' for the “Robotics Kit”.

Once your school has a site licence, you may treat this work as you like.

It may be modified, cut up to make 'work sheets' and passed on to interested colleagues and students. ( You may USE this material as if it were yours, but I would get upset if you CLAIMED it as yours!)

This project has been called kit number "ONE" for good reason. With a little funding support, I hope to come up with faster, more accurate sensing techniques, making more use of 'assembled' routines and AtoD interfaces. Purchase of site rights for this kit will ensure continuing support from me as more material is produced (and provide the funds needed for components and software!).

NOTE: This is a fund-raising project for the school. ALL funds raised by the sale of site licences are paid into school funds!

Good Luck.

Regards,

Jim Fuller.